Energy Controller System

(Using ESP32 & PZEM004T with TFT Display)

User Manual

| Filling Reference : | Integra/AMU/KIT2/User Manual |  |
| --- | --- | --- |
| Document Title : | User Manual |  |
| Version : | Draft |  |
| Document Author: | Mr. Sabarish |  |
| Date Created : | 01/06/2025 |  |
| Date Modified : | 01/06/2025 |  |

|  | Reviewed By | Accepted By |
| --- | --- | --- |
| Signature & Date |  |  |
| Name | Mr. Bala |  |
| Designation | Technical Consultant |  |

Change History:

| Version | Date | Author | Change History |
| --- | --- | --- | --- |
| Draft | 01/06/2025 | Mr. Sabarish | Initial Draft Created |

Distribution List:

| Name | Email Address |
| --- | --- |
| Mr. Bala | bala@integratech.com.sg |
| Mr. Hari | hari@integratech.com.sg |
| Mr. Selva Pandian | selvapandian@integratech.com.sg |
| Mr. Sabarish | sabarish@integratech.com.sg |
| Mr. Sivaguru | sivaguru@integratech.com.sg |

Table of Contents

| Contents 1. Project Overview 4 2. Objective 4 3. Hardware Component 5 4. Software Component 6 5. Circuit Diagram and Wiring 7 6. Code Explanation 10 7. Setup and Usage Instructions 13 8. Data Flow and Communication 15 9. Trouble shooting and FAQs 16 10. Code Examples 17 Class 1: WiFiManager + OTA Hansdler 17 Class 2: TFT Display Handler 18 Class 3: PZEM Sensor Handler 19 Class 4: Relay Handler 20 Class 5: ThingsBoard + Flask HTTP Handler 21 |  |
| --- | --- |

- Project Overview

The Energy Controller is a robust IoT-based system engineered to monitor key electrical parameters in real time and manage power delivery through relay control. At its core is the ESP32 microcontroller, seamlessly integrated with the PZEM-004T v3.0 energy monitoring module and a 2.2” TFT display for on-device data visualization. The system transmits telemetry data to the ThingsBoard IoT platform and a backend server powered by a Flask API with MongoDB for persistent storage. It also supports over-the-air (OTA) firmware updates and enables remote relay control, offering a comprehensive solution for smart energy management.

- Objective

- Monitor key electrical parameters including voltage, current, power, frequency, and power factor of connected loads.

- Display real-time measurements on a 2.2” TFT screen for immediate local visualization.

- Securely transmit telemetry data to both the ThingsBoard IoT platform and a Flask-based backend API.

- Enable remote relay control through ThingsBoard, allowing users to manage power delivery from anywhere.

- Support seamless firmware updates via Over-the-Air (OTA) functionality.

- Simplify network configuration using WiFiManager’s captive portal for intuitive Wi-Fi setup.

- Hardware Component

| Component | Description | Quantity |
| --- | --- | --- |
| ESP32 Development Board | Main microcontroller | 1 |
| PZEM-004T v3.0 | Energy monitor sensor module | 1 |
| 2.2" TFT Display (ILI9225) | SPI TFT display for data visualization | 1 |
| Relay Module | Controls external device ON/OFF | 1 |
| Connecting Wires | Jumper wires for connections | As needed |
| Power Supply | 5V or regulated power source | 1 |

- Software Component

| Software/Library | Purpose |
| --- | --- |
| Arduino IDE | Main microcontroller with Wi-Fi capability |
| SPI.h | SPI communication |
| TFT_22_ILI9225.h | TFT display driver |
| FreeSans9pt7b.h | Font for display |
| PZEM004Tv30.h | Library for PZEM sensor |
| WiFi.h | WiFi connectivity |
| WiFiManager.h | WiFi auto-configuration |
| ArduinoOTA.h | OTA firmware updates |
| HTTPClient.h | HTTP client for API calls |
| WiFiClientSecure.h | Secure HTTP client |
| ArduinoJson.h | JSON parsing and serialization |

- Circuit Diagram and Wiring

Pin Assignments

| Device | ESP32 Pin | Signal Description |
| --- | --- | --- |
| TFT Display | GPIO26 (TFT_RST) | Reset |
|  | GPIO25 (TFT_RS) | Register Select / Data Cmd |
|  | GPIO15 (TFT_CS) | Chip Select |
|  | GPIO13 (TFT_SDI) | SPI MOSI |
|  | GPIO14 (TFT_CLK) | SPI Clock |
|  | GPIO0 (TFT_LED) | Backlight control |
| Relay | GPIO4 | Relay control signal |
| PZEM-004T | GPIO16 (RX2) | UART2 RX (ESP32 receives) |
|  | GPIO17 (TX2) | UART2 TX (ESP32 transmits) |

Wiring Instructions

- TFT Display: Connect the TFT display pins to the ESP32 according to the specified pin mapping.

- Relay Module: Connect the relay input (IN) to GPIO4 on the ESP32. Ensure the relay’s power and ground lines are connected to a 5V supply and GND, respectively.

- PZEM-004T Sensor: Connect the TX pin of the PZEM to GPIO16 (RX2) and the RX pin to GPIO17 (TX2) on the ESP32.

- Grounding: Use a common ground across all components to ensure stable operation.

- Power Supply: Provide a 5V regulated power source (or as specified) to power the PZEM module and the TFT display backlight.

ESP32

GPIO26 (RST) -----------------> TFT RST GPIO25 (RS/DC) -----------------> TFT RS GPIO15 (CS) ------------------>TFT CS GPIO13 (MOSI) ------------------> TFT SDI (MOSI) GPIO14 (SCK) ------------------>TFT CLK (SCK) GPIO0 (LED) ------------------> TFT LED (Backlight)

RELAY

GPIO4 -------------------------> Relay IN

PZEM

GPIO16 (RX2) <----------------- PZEM TX GPIO17 (TX2) -----------------> PZEM RX

All grounds connected together

Power:

- TFT and PZEM powered by 5V (or 3.3V as per specs)

- ESP32 powered via USB or 3.3V supply

- Code Explanation

Initialization

- Serial Ports:

  - Serial is initialized for debugging at 115200 baud.

  - Serial2 is configured for communication with the PZEM-004T sensor at 9600 baud.

- TFT Display:

  - Utilizes SPI communication via hardware HSPI pins. The display is initialized with the FreeSans9pt7b font for improved readability.

- Relay Setup:

  - Configures GPIO4 as an output pin. The relay is set to OFF by default.

- WiFiManager:

  - Launches a captive portal if Wi-Fi credentials are not already stored, allowing easy network configuration.

- OTA (Over-the-Air Updates):

  - Initializes the OTA service to support remote firmware updates.

Main Loop

- OTA Handler:

  - Continuously runs to keep the OTA service active.

- Sensor Reading:

  - Retrieves real-time values for voltage, current, power, frequency, and power factor from the PZEM sensor.

- Display Update:

  - Clears the relevant area on the TFT screen and displays the latest sensor readings along with the relay status.

- Data Upload:

  - Sends telemetry data and relay status as JSON to ThingsBoard via HTTPS.

  - Sends similar data to the Flask API via HTTP.

- Relay Control:

  - Invokes the controlRelay() function to fetch the current relay state from ThingsBoard and update the output pin accordingly.

- Delay:

  - Introduces a 1-second delay before the next iteration.

Relay Control Function

- Sends a secure GET request to the ThingsBoard API to retrieve the current relay status.

- Parses the JSON response to determine if "relay": true or "relay": false.

- Updates the relay output pin and the local relayStatus variable based on the response.

- Logs the status or any error messages to the Serial Monitor.

- Setup and Usage Instructions

Hardware Setup

- Connect all components according to the wiring diagram.

- Power the ESP32 using a USB cable or a regulated 3.3V power supply.

- Ensure the PZEM-004T sensor is connected in series with the load line you intend to monitor.

Always observe proper electrical safety precautions.

Software Setup

- Open the Arduino IDE.

- Install the following required libraries:

  - TFT_22_ILI9225

  - PZEM004Tv30

  - WiFiManager

  - ArduinoOTA

  - ArduinoJson

- Update the following variables in the code with your actual server details:

  - kitID

  - thingsboardURL

  - thingsboardRelayURL

  - flaskAPI

- Upload the code to the ESP32.

- On first boot, the ESP32 will create a Wi-Fi access point named "Energy-Controller".

- Connect your phone or computer to this access point and configure Wi-Fi credentials via the captive portal.

- Once configured, the ESP32 will connect to the specified Wi-Fi network and begin operation.

Monitoring

- View real-time electrical parameters on the TFT display.

- Monitor system logs and sensor data via the Serial Monitor.

- Access the ThingsBoard dashboard to visualize data and control the relay remotely.

- Use the Flask backend to review and manage stored telemetry data.

OTA Firmware Update

- Ensure both the ESP32 and your computer are connected to the same Wi-Fi network.

- Use the Arduino IDE’s OTA upload feature to update the firmware.

- The ESP32’s OTA hostname is: "Energy-Controller".

- Data Flow  and Communication

Sensor Data Acquisition

- The PZEM-004T sensor transmits electrical measurements to the ESP32 via UART2.

- The ESP32 reads the following parameters:

  - Voltage

  - Current

  - Power

  - Frequency

  - Power Factor

Data Display

- Real-time sensor data is displayed locally on the TFT screen.

- The display layout is clean and labeled for easy readability.

Data Transmission

- The ESP32 packages the sensor data and relay status into JSON format.

- This data is transmitted to:

  - ThingsBoard via HTTPS at api/v1/esp2/telemetry

  - Flask REST API via HTTP at /api/v1/data

Relay Control

- The ESP32 sends a GET request to the ThingsBoard attribute API to retrieve the current relay state.

- It parses the JSON response to determine the relay command (true or false).

- Based on the response, the ESP32 updates the relay output pin accordingly.

- Trouble shooting and FAQs

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| TFT display shows nothing | Incorrect wiring or no backlight | Verify SPI pins, power, and backlight pin |
| No sensor data or NAN values | PZEM wiring or serial config problem | Check RX/TX connections; use logic analyzer |
| WiFi connection fails | Incorrect credentials or weak signal | Use captive portal to reset WiFi settings |
| OTA update fails | Firewall or network issue | Ensure device and PC are on same network |
| Relay does not switch | Wrong pin or relay logic inverted | Verify GPIO4 connection and relay logic |
| ThingsBoard POST fails | Wrong URL or certificate error | Confirm server IP, port, and use setInsecure() for local certs |

- Code Examples

## Class 1: WiFiManager + OTA Hansdler

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

class WifiOTAHandler {

public:

WifiOTAHandler(const char* apName = "Energy-Controller") : apName(apName) {}

void begin() {

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect(apName)) {

Serial.println("Failed to connect WiFi, restarting...");

ESP.restart();

}

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.setHostname(apName);

ArduinoOTA.begin();

}

void handle() {

ArduinoOTA.handle();

}

private:

const char* apName;

};

OUTPUT:

[WiFiManager] Attempting to connect...

*WM: AutoConnect

*WM: Connecting as wifi client...

*WM: Connection result: SUCCESS

WiFi connected: 192.168.1.101

[OTA] OTA initialized with hostname: Energy-Controller

## Class 2: TFT Display Handler

#include "SPI.h"

#include "TFT_22_ILI9225.h"

#include <../fonts/FreeSans9pt7b.h>

class TFTHandler {

public:

TFTHandler() :

tft(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS) {}

void begin() {

hspi.begin();

tft.begin(hspi);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

}

void displayText(int x, int y, const char* text, uint16_t color = COLOR_WHITE) {

tft.drawGFXText(x, y, text, color);

}

void clearArea(int x, int y, int w, int h) {

tft.fillRectangle(x, y, w, h, COLOR_BLACK);

}

void displayIP(IPAddress ip) {

char ipStr[16];

sprintf(ipStr, "%d.%d.%d.%d", ip[0], ip[1], ip[2], ip[3]);

clearArea(0, 28, 176, 20);

displayText(10, 50, ipStr, COLOR_WHITE);

}

private:

static constexpr int TFT_RST = 26;

static constexpr int TFT_RS = 25;

static constexpr int TFT_CS = 15;

static constexpr int TFT_LED = 0;

static constexpr int TFT_BRIGHTNESS = 200;

SPIClass hspi{HSPI};

TFT_22_ILI9225 tft;

};

OUTPUT:

[TFT] Initializing TFT screen...

[TFT] Display cleared.

[TFT] Font set.

[TFT] Displaying text: "System Ready" at (10, 30)

[TFT] Displaying IP address: 192.168.1.101

## Class 3: PZEM Sensor Handler

#include <PZEM004Tv30.h>

class PZEMHandler {

public:

PZEMHandler(HardwareSerial &serial, int rxPin, int txPin) : pzem(&serial, rxPin, txPin) {}

void begin() {

pzemSerial->begin(9600, SERIAL_8N1, rxPin, txPin);

}

float getVoltage() { return pzem.voltage(); }

float getCurrent() { return pzem.current(); }

float getPower()   { return pzem.power(); }

private:

PZEM004Tv30 pzem;

HardwareSerial* pzemSerial = &Serial2;

int rxPin = 16;

int txPin = 17;

};

OUTPUT:

[PZEM] Initializing sensor...

[PZEM] Reading data...

[PZEM] Voltage: 227.30 V

[PZEM] Current: 0.47 A

[PZEM] Power: 106.83 W

## Class 4: Relay Handler

class RelayHandler {

public:

RelayHandler(int pin) : relayPin(pin), state(false) {}

void begin() {

pinMode(relayPin, OUTPUT);

digitalWrite(relayPin, LOW);

}

void setState(bool on) {

state = on;

digitalWrite(relayPin, state ? HIGH : LOW);

}

bool getState() {

return state;

}

void toggle() {

setState(!state);

}

private:

int relayPin;

bool state;

};

OUTPUT:

[Relay] Initializing relay on pin 12...

[Relay] Current state: OFF

[Relay] Toggling relay...

[Relay] Relay state is now: ON

[Relay] Setting relay OFF...

[Relay] Relay state is now: OFF

## Class 5: ThingsBoard + Flask HTTP Handler

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <ArduinoJson.h>

class HttpHandler {

public:

HttpHandler(const char* thingsboardURL, const char* relayURL, const char* flaskURL) :

tbURL(thingsboardURL), relayURL(relayURL), flaskURL(flaskURL) {}

void sendTelemetry(float voltage, float current, float power) {

WiFiClientSecure client;

client.setInsecure();

HTTPClient https;

if (https.begin(client, tbURL)) {

https.addHeader("Content-Type", "application/json");

StaticJsonDocument<256> doc;

doc["voltage"] = voltage;

doc["current"] = current;

doc["power"] = power;

String json;

serializeJson(doc, json);

int code = https.POST(json);

Serial.printf("ThingsBoard Telemetry POST code: %d\n", code);

https.end();

}

}

bool fetchRelayState() {

WiFiClientSecure client;

client.setInsecure();

HTTPClient https;

bool relayStatus = false;

if (https.begin(client, relayURL)) {

int code = https.GET();

if (code == 200) {

String payload = https.getString();

StaticJsonDocument<256> doc;

DeserializationError err = deserializeJson(doc, payload);

if (!err) {

relayStatus = doc["relay"] | false;

}

}

https.end();

}

return relayStatus;

}

void sendFlaskData(const char* kitID, float voltage, float current, float power, bool relay) {

HTTPClient http;

if (http.begin(flaskURL)) {

http.addHeader("Content-Type", "application/json");

StaticJsonDocument<256> doc;

doc["kitID"] = kitID;

doc["voltage"] = voltage;

doc["current"] = current;

doc["power"] = power;

doc["relay"] = relay;

String json;

serializeJson(doc, json);

int code = http.POST(json);

Serial.printf("Flask POST code: %d\n", code);

http.end();

}

}

private:

const char* tbURL;

const char* relayURL;

const char* flaskURL;

};

OUTPUT:

[HTTP] Connecting to ThingsBoard...

[HTTP] Sending telemetry:

{

"voltage": 227.30,

"current": 0.47,

"power": 106.83

}

ThingsBoard Telemetry POST code: 200

🔄 Fetching relay state from remote server:

[HTTP] Fetching relay state from relayURL...

[HTTP] Response:

{

"relay": true

}

[HTTP] Relay state received: ON

📤 Sending data to Flask server:

[HTTP] Sending data to Flask:

{

"kitID": "KIT001",

"voltage": 227.30,

"current": 0.47,

"power": 106.83,

"relay": true

}

Flask POST code: 200s