Air Quality Monitoring System

(Using ESP32, DHT11, MQ7, MQ135, SDS011 & PZEM004T TFT Display)

User Manual

| Filling Reference : | Integra/AMU/KIT5/User Manual |  |
| --- | --- | --- |
| Document Title : | User Manual |  |
| Version : | Draft |  |
| Document Author: | Mr. Sabarish |  |
| Date Created : | 01/06/2025 |  |
| Date Modified : | 01/06/2025 |  |

|  | Reviewed By | Accepted By |
| --- | --- | --- |
| Signature & Date |  |  |
| Name | Mr. Bala |  |
| Designation | Technical Consultant |  |

Change History:

| Version | Date | Author | Change History |
| --- | --- | --- | --- |
| Draft | 01/06/2025 | Mr. Sabarish | Initial Draft Created |

Distribution List:

| Name | Email Address |
| --- | --- |
| Mr. Bala | bala@integratech.com.sg |
| Mr. Hari | hari@integratech.com.sg |
| Mr. Selva Pandian | selvapandian@integratech.com.sg |
| Mr. Sabarish | sabarish@integratech.com.sg |
| Mr. Sivaguru | sivaguru@integratech.com.sg |

Table of Contents

| Contents 1. Project Overview 4 2. Objectives 4 3. Hardware Component 5 4. Software Component 5 5. Circuit Diagram and Wiring 6 6. Code Explanation 9 7. Setup and Usage Instructions 10 8. Data Flow and Communication 12 9. Troubleshooting and FAQs 13 10. Sample Code 15 Class 1: DHT11 + TFT 15 Class 2: CO (MQ7) + TFT 16 Class 3: MQ135 (Air Quality) + TFT 17 Class 4: PM2.5 & PM10 (SDS011) + TFT 18 Class 5: DHT + MQ7 (CO) + TFT 19 Class 6: DHT + MQ135 (Air Q) + TFT 20 Class 7: Full System (All Sensors) + TFT 21 |  |
| --- | --- |

- Project Overview

This project is an Air Quality Monitoring System powered by an ESP32 microcontroller. It utilizes multiple sensors to track environmental parameters, including temperature, humidity, carbon monoxide (CO) levels, overall air quality (via MQ135), and particulate matter concentrations (PM2.5 and PM10). The real-time data is displayed on a TFT screen and transmitted to both the Things Board IoT platform and a custom Flask API, enabling remote monitoring and data analysis. This setup ensures comprehensive air quality tracking and provides valuable insights for environmental awareness and safety.

- Objectives

- Purpose: Monitor air quality by measuring temperature, humidity, CO, general air quality, PM2.5, and PM10.

- Display: Real-time data shown on a 2.2" TFT screen.

- Connectivity: Sends data to ThingsBoard (IoT platform) and a custom Flask API.

- Features: OTA updates, WiFiManager for easy setup, JSON telemetry.

- Hardware Component

- ESP32: Core microcontroller.

- Sensors:

  - DHT11: Temperature & humidity.

  - MQ7: Carbon monoxide.

  - MQ135: General air quality.

  - SDS011: PM2.5 and PM10.

- Display: 2.2" TFT (ILI9225).

- Power: 3.3V regulated supply.

- Miscellaneous: Jumper wires, PCB for connections

- Software Component

- Development: Arduino IDE / PlatformIO.

- Libraries: SDS011, DHT, WiFiManager, ArduinoOTA, HTTPClient, ArduinoJson, TFT_22_ILI9225.

- Platforms: ThingsBoard, Flask API.

- Circuit Diagram and Wiring

Key Connector

| Sensor | ESP32 Pin | Description |
| --- | --- | --- |
| SDS011 TX | GPIO 17 (Serial1 RX) | Serial1 RX on ESP32 |
| SDS011 RX | GPIO 16 (Serial1 TX) | Serial1 TX on ESP32 |
| DHT11 Data | GPIO 32 | Single wire data |
| MQ7 Analog | GPIO 34 | ADC input |
| MQ135 Analog | GPIO 35 | ADC input |
| TFT Display |  | SPI Bus on HSPI (GPIOs 13,14,15,25,26) |
| TFT_RST | GPIO 26 | Reset pin for TFT |
| TFT_RS | GPIO 25 | Register select |
| TFT_CS | GPIO 15 | Chip select |
| TFT_SDI (MOSI) | GPIO 13 | SPI MOSI |
| TFT_CLK (SCK) | GPIO 14 | SPI Clock |
| TFT_LED | GPIO 0 | Backlight control |

- Code Explanation

Setup()

- Initializes Serial for debugging.

- Initializes sensors:

- DHT11 (Temperature & Humidity)

- MQ7 (Carbon Monoxide)

- MQ135 (Air Quality)

- SDS011 (Particulate Matter)

- Sets up SPI communication with the TFT display and initializes it.

- Displays the project title on the TFT screen.

- Uses WiFiManager to manage WiFi credentials (sets up Access Point if no configuration is found).

- Starts OTA (Over-The-Air) updates for remote firmware upgrades.

Loop()

- Handles OTA updates.

- Reads raw sensor values:

  - MQ7 and MQ135 analog readings are converted to gas concentrations using sensor-specific calibration and formulas.

  - Reads temperature and humidity from the DHT11 sensor.

  - Reads PM2.5 and PM10 values from the SDS011 sensor.

- Applies calibration factors to the sensor data.

- Displays all sensor readings on the TFT display with appropriate labels and units.

- Sends JSON-formatted telemetry data to:

  - ThingsBoard (via secure HTTPS connection)

  - Flask API (via HTTP connection)

- Waits for 5 seconds before the next cycle.

- Setup and Usage Instructions

Hardware Setup

- Connect all sensors and the TFT display to the ESP32 according to the pin mapping provided earlier.

- Ensure a stable 3.3V power supply is available for both the ESP32 and the connected sensors.

- Connect the ESP32 to your computer using a USB cable.

Software Setup

- Install the Arduino IDE or Platform IO as your development environment.

- In the Arduino IDE, open the Library Manager and install the following libraries:

  - SDS011 – for the particulate matter sensor

  - DHT sensor library – for temperature and humidity

  - TFT_22_ILI9225 – for TFT display control

  - WiFiManager – for WiFi configuration

  - ArduinoJson – for JSON formatting

  - ArduinoOTA – for OTA firmware updates

  - HTTPClient – for HTTP communication

- Modify the code to include your:

  - ThingsBoard server URL and access token

  - Flask API endpoint URL

- Upload the modified code to the ESP32 using the Arduino IDE or PlatformIO.

First Boot Instructions

- Upon first boot, the ESP32 will create a Wi-Fi Access Point named "AirMonitor-AP".

- Connect to this AP using a smartphone or PC.

- A captive portal will open, prompting you to enter your Wi-Fi credentials.

- Once configured, the device will connect to your WiFi network and begin monitoring air quality.

- Data Flow and Communication

The system follows a structured flow of data from sensor input to cloud and local server communication:

Sensor Input

→ Environmental sensors (DHT11, MQ7, MQ135, SDS011) send analog/digital signals

→ ESP32 receives and processes the data.

Local Display

→ Processed sensor data is displayed on the TFT screen in real time.

Cloud Communication

→ ESP32 sends JSON-formatted telemetry via HTTPS to the ThingsBoard IoT platform for remote monitoring.

Local Server Communication

→ ESP32 also sends the same telemetry via HTTP to a custom Flask API for local data logging or visualization.

Firmware Updates

→ OTA (Over-The-Air) updates are supported, allowing firmware upgrades over WiFi without physical access to the device.

- Troubleshooting and FAQs

❓ Q: The TFT screen does not display anything.

A:

- Check the SPI wiring: CLK, MOSI, CS, RS, and RST.

- Ensure the power and backlight (LED) pins are properly connected.

- Confirm that the hspi.begin() configuration matches your hardware pin setup.

❓ Q: Sensors give zero or constant readings.

A:

- Verify that the sensors are powered and correctly wired.

- For MQ sensors, ensure they are preheated properly (they require a burn-in period).

- Check that the ADC pins (e.g., GPIO34, GPIO35) are correct and not left floating.

❓ Q: The device cannot connect to WiFi.

A:

- Reset the device to trigger WiFiManager AP mode.

- Connect to the "AirMonitor-AP" and reconfigure WiFi credentials.

- Check your WiFi signal strength and router settings.

❓ Q: OTA (Over-The-Air) updates fail.

A:

- Ensure the ESP32 and host PC are on the same network.

- Confirm the OTA hostname is correctly set in the code.

- Use the Arduino IDE and select the correct OTA port.

❓ Q: HTTP POST requests to the server fail.

A:

- Verify the server URLs and port numbers in the code.

- Check for network firewalls or SSL certificate issues.

- For local servers with self-signed certificates, use client.setInsecure() in the code.

- Sample Code

## Class 1: DHT11 + TFT

Code

#include <DHT.h>
#include "SPI.h"
#include "TFT_22_ILI9225.h"
#include <../fonts/FreeSans9pt7b.h>

// Define DHT11 sensor pin and type
#define DHTPIN 32
#define DHTTYPE DHT11
DHT dht(DHTPIN, DHTTYPE);

// TFT Setup (assumes common SPI configuration block is defined elsewhere)

void setup() {
  Serial.begin(115200);
  dht.begin();

  hspi.begin(TFT_CLK, -1, TFT_SDI, TFT_CS);
  tft.begin(hspi);
  tft.setOrientation(0);
  tft.clear();
  tft.setGFXFont(&FreeSans9pt7b);
  tft.drawGFXText(10, 20, "DHT11 Monitor", COLOR_YELLOW);
}

void loop() {
  float temp = dht.readTemperature();
  float hum = dht.readHumidity();

  tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);
  tft.drawGFXText(5, 45, "Temp:", COLOR_CYAN);
  tft.drawGFXText(100, 45, String(temp, 1) + " C", COLOR_WHITE);

  tft.drawGFXText(5, 69, "Humidity:", COLOR_CYAN);
  tft.drawGFXText(100, 69, String(hum, 1) + " %", COLOR_WHITE);

  delay(5000); // Wait 5 seconds before next reading
}

TFT Output Example

DHT11 Monitor
------------------------
Temp:      28.5 C
Humidity:  62.0 %

## Class 2: CO (MQ7) + TFT

Code

#include <DHT.h>
#include "SPI.h"
#include "TFT_22_ILI9225.h"
#include <../fonts/FreeSans9pt7b.h>

// Define MQ7 sensor pin and constants
#define MQ7_PIN 34
#define RL_MQ7 10000.0
float R0_MQ7 = 17933.0;
float calFactor_CO = 0.0787;

// TFT Setup (assumes common SPI configuration block is defined elsewhere)

void setup() {
  Serial.begin(115200);
  pinMode(MQ7_PIN, INPUT);

  hspi.begin(TFT_CLK, -1, TFT_SDI, TFT_CS);
  tft.begin(hspi);
  tft.setOrientation(0);
  tft.clear();
  tft.setGFXFont(&FreeSans9pt7b);
  tft.drawGFXText(10, 20, "CO Monitor", COLOR_YELLOW);
}

void loop() {
  int raw = analogRead(MQ7_PIN);
  float Vout = raw * (3.3 / 4095.0);
  float RS = (3.3 - Vout) * RL_MQ7 / Vout;
  float ratio = RS / R0_MQ7;
  float ppm = pow(10, ((log10(ratio) - 0.8) / -0.38));
  float calibrated = ppm * calFactor_CO;

  tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);
  tft.drawGFXText(5, 45, "CO:", COLOR_CYAN);
  tft.drawGFXText(100, 45, String(calibrated, 2) + " ppm", COLOR_WHITE);

  delay(5000); // Wait 5 seconds before next reading
}

TFT Output Example

TFT Output:
CO Monitor
------------------------
CO:        1.37 ppm

## Class 3: MQ135 (Air Quality) + TFT

Code
#include <DHT.h>
#include "SPI.h"
#include "TFT_22_ILI9225.h"
#include <../fonts/FreeSans9pt7b.h>

// Define MQ135 sensor pin and constants
#define MQ135_PIN 35
#define RL_MQ135 10000.0
float R0_MQ135 = 28408.0;

// TFT Setup (assumes common SPI configuration block is defined elsewhere)

void setup() {
  Serial.begin(115200);
  pinMode(MQ135_PIN, INPUT);

  hspi.begin(TFT_CLK, -1, TFT_SDI, TFT_CS);
  tft.begin(hspi);
  tft.setOrientation(0);
  tft.clear();
  tft.setGFXFont(&FreeSans9pt7b);
  tft.drawGFXText(10, 20, "Air Quality", COLOR_YELLOW);
}

void loop() {
  int raw = analogRead(MQ135_PIN);
  float Vout = raw * (3.3 / 4095.0);
  float RS = (3.3 - Vout) * RL_MQ135 / Vout;
  float ratio = RS / R0_MQ135;
  float ppm = 116.6020682 * pow(ratio, -2.769034857);

  tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);
  tft.drawGFXText(5, 45, "Air Q:", COLOR_CYAN);
  tft.drawGFXText(100, 45, String(ppm, 2) + " ppm", COLOR_WHITE);

  delay(5000);
}

TFT Output Example

Air Quality
------------------------
Air Q:     152.66 ppm

## Class 4: PM2.5 & PM10 (SDS011) + TFT

#include <SDS011.h>

SDS011 sds;
float calFactor_PM25 = 0.79;
float calFactor_PM10 = 1.61;

void setup() {
  Serial.begin(115200);
  Serial1.begin(9600, SERIAL_8N1, 17, 16);  // RX=17, TX=16
  sds.begin(&Serial1);

  hspi.begin(TFT_CLK, -1, TFT_SDI, TFT_CS);
  tft.begin(hspi);
  tft.setOrientation(0);
  tft.clear();
  tft.setGFXFont(&FreeSans9pt7b);
  tft.drawGFXText(10, 20, "PM Sensor", COLOR_YELLOW);
}

void loop() {
  float pm25 = 0, pm10 = 0;
  if (sds.read(&pm25, &pm10) != 0) {
    pm25 = 0;
    pm10 = 0;
  }
  float calibrated25 = pm25 * calFactor_PM25;
  float calibrated10 = pm10 * calFactor_PM10;

  tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);
  tft.drawGFXText(5, 45, "PM2.5:", COLOR_CYAN);
  tft.drawGFXText(100, 45, String(calibrated25, 1) + " ug", COLOR_WHITE);
  tft.drawGFXText(5, 69, "PM10:", COLOR_CYAN);
  tft.drawGFXText(100, 69, String(calibrated10, 1) + " ug", COLOR_WHITE);

  delay(5000);
}

TFT Output:
PM Sensor
------------------------
PM2.5:     18.4 ug
PM10:      56.2 ug

## Class 5: DHT + MQ7 (CO) + TFT

#include <DHT.h>

#define MQ7_PIN 34
#define RL_MQ7 10000.0
float R0_MQ7 = 17933.0;
float calFactor_CO = 0.0787;

#define DHTPIN 32
#define DHTTYPE DHT11
DHT dht(DHTPIN, DHTTYPE);

// TFT Setup (use common block above)

void setup() {
  Serial.begin(115200);
  dht.begin();
  pinMode(MQ7_PIN, INPUT);

  hspi.begin(TFT_CLK, -1, TFT_SDI, TFT_CS);
  tft.begin(hspi);
  tft.setOrientation(0);
  tft.clear();
  tft.setGFXFont(&FreeSans9pt7b);
  tft.drawGFXText(10, 20, "DHT + CO", COLOR_YELLOW);
}

void loop() {
  float temp = dht.readTemperature();
  float hum = dht.readHumidity();

  int raw = analogRead(MQ7_PIN);
  float Vout = raw * (3.3 / 4095.0);
  float RS = (3.3 - Vout) * RL_MQ7 / Vout;
  float ratio = RS / R0_MQ7;
  float ppm = pow(10, ((log10(ratio) - 0.8) / -0.38));
  float calibrated = ppm * calFactor_CO;

  tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);
  int y = 45, spacing = 24;
  tft.drawGFXText(5, y, "Temp:", COLOR_CYAN); tft.drawGFXText(100, y, String(temp, 1) + " C", COLOR_WHITE); y += spacing;
  tft.drawGFXText(5, y, "Humidity:", COLOR_CYAN); tft.drawGFXText(100, y, String(hum, 1) + " %", COLOR_WHITE); y += spacing;
  tft.drawGFXText(5, y, "CO:", COLOR_CYAN); tft.drawGFXText(100, y, String(calibrated, 2) + " ppm", COLOR_WHITE);

  delay(5000);
}

TFT Output:

DHT + CO
------------------------
Temp:      27.9 C
Humidity:  59.0 %
CO:        1.52 ppm

## Class 6: DHT + MQ135 (Air Q) + TFT

#include <DHT.h>

#define MQ135_PIN 35

#define RL_MQ135 10000.0

float R0_MQ135 = 28408.0;

#define DHTPIN 32

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

// TFT Setup (use common block above)

void setup() {

Serial.begin(115200);

dht.begin();

pinMode(MQ135_PIN, INPUT);

hspi.begin(TFT_CLK, -1, TFT_SDI, TFT_CS);

tft.begin(hspi);

tft.setOrientation(0);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 20, "DHT + AQ", COLOR_YELLOW);

}

void loop() {

float temp = dht.readTemperature();

float hum = dht.readHumidity();

int raw = analogRead(MQ135_PIN);

float Vout = raw * (3.3 / 4095.0);

float RS = (3.3 - Vout) * RL_MQ135 / Vout;

float ratio = RS / R0_MQ135;

float ppm = 116.6020682 * pow(ratio, -2.769034857);

tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);

int y = 45, spacing = 24;

tft.drawGFXText(5, y, "Temp:", COLOR_CYAN); tft.drawGFXText(100, y, String(temp, 1) + " C", COLOR_WHITE); y += spacing;

tft.drawGFXText(5, y, "Humidity:", COLOR_CYAN); tft.drawGFXText(100, y, String(hum, 1) + " %", COLOR_WHITE); y += spacing;

tft.drawGFXText(5, y, "Air Q:", COLOR_CYAN); tft.drawGFXText(100, y, String(ppm, 2) + " ppm", COLOR_WHITE);

delay(5000);

}

TFT Output:

DHT + AQ

------------------------

Temp:      26.7 C

Humidity:  60.5 %

Air Q:     147.92 ppm

## Class 7: Full System (All Sensors) + TFT

#include <SDS011.h>

#include <DHT.h>

#include <math.h>

#include "SPI.h"

#include "TFT_22_ILI9225.h"

#include <../fonts/FreeSans9pt7b.h>

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <ArduinoJson.h>

// === TFT Setup ===

#define TFT_RST 26

#define TFT_RS  25

#define TFT_CS  15

#define TFT_SDI 13

#define TFT_CLK 14

#define TFT_LED 0

#define TFT_BRIGHTNESS 200

SPIClass hspi(HSPI);

TFT_22_ILI9225 tft = TFT_22_ILI9225(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS);

// === Sensor Setup ===

#define MQ7_PIN 34

#define MQ135_PIN 35

#define DHTPIN 32

#define DHTTYPE DHT11

#define RL_MQ7 10000.0

#define RL_MQ135 10000.0

float R0_MQ7 = 17933.0;

float R0_MQ135 = 28408.0;

float calFactor_CO   = 0.0787;

float calFactor_PM25 = 0.79;

float calFactor_PM10 = 1.61;

DHT dht(DHTPIN, DHTTYPE);

SDS011 my_sds;

// === Network and API Setup ===

const char* kitID = "kit5";

const char* thingsboardURL = "https://192.168.1.238:8080/api/v1/esp5/telemetry";

const char* flaskAPI = "http://192.168.1.130:5010/api/v1/data";

void setup() {

Serial.begin(115200);

// === Sensor Setup ===

dht.begin();

pinMode(MQ7_PIN, INPUT);

pinMode(MQ135_PIN, INPUT);

Serial1.begin(9600, SERIAL_8N1, 17, 16);

my_sds.begin(&Serial1);

// === TFT Setup ===

hspi.begin(TFT_CLK, -1, TFT_SDI, TFT_CS);

tft.begin(hspi);

tft.setOrientation(0);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 20, "Air Quality Monitor", COLOR_YELLOW);

// === WiFi Manager ===

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("AirMonitor-AP")) {

Serial.println("WiFi failed. Restarting...");

ESP.restart();

}

Serial.println("WiFi connected: " + WiFi.localIP().toString());

// === OTA Setup ===

ArduinoOTA.setHostname("AirMonitor");

ArduinoOTA.begin();

}

void loop() {

ArduinoOTA.handle();

// === Read Sensors ===

int rawMQ7 = analogRead(MQ7_PIN);

float Vout_MQ7 = rawMQ7 * (3.3 / 4095.0);

float RS_MQ7 = (3.3 - Vout_MQ7) * RL_MQ7 / Vout_MQ7;

float ratio_MQ7 = RS_MQ7 / R0_MQ7;

float ppm_CO = pow(10, ((log10(ratio_MQ7) - 0.8) / -0.38));

float calibrated_CO = ppm_CO * calFactor_CO;

int rawMQ135 = analogRead(MQ135_PIN);

float Vout_MQ135 = rawMQ135 * (3.3 / 4095.0);

float RS_MQ135 = (3.3 - Vout_MQ135) * RL_MQ135 / Vout_MQ135;

float ratio_MQ135 = RS_MQ135 / R0_MQ135;

float ppm_air = 116.6020682 * pow(ratio_MQ135, -2.769034857);

float temp = dht.readTemperature();

float hum = dht.readHumidity();

float pm25 = 0, pm10 = 0;

if (my_sds.read(&pm25, &pm10) != 0) {

pm25 = 0;

pm10 = 0;

}

float calibrated_PM25 = pm25 * calFactor_PM25;

float calibrated_PM10 = pm10 * calFactor_PM10;

// === Display on TFT ===

tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);

int16_t labelX = 5, valueX = 100, y = 45, spacing = 24;

tft.drawGFXText(labelX, y, "Temp:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(temp, 1) + " C", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "Humidity:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(hum, 1) + " %", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "CO:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(calibrated_CO, 2) + " ppm", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "Air Q:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(ppm_air, 2) + " ppm", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "PM2.5:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(calibrated_PM25, 1) + " ug", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "PM10:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(calibrated_PM10, 1) + " ug", COLOR_WHITE);

// === Send to ThingsBoard ===

WiFiClientSecure client;

client.setInsecure();

HTTPClient https;

if (https.begin(client, thingsboardURL)) {

https.addHeader("Content-Type", "application/json");

StaticJsonDocument<256> doc;

doc["temperature"] = temp;

doc["humidity"] = hum;

doc["co"] = calibrated_CO;

doc["air_quality"] = ppm_air;

doc["pm25"] = calibrated_PM25;

doc["pm10"] = calibrated_PM10;

String json;

serializeJson(doc, json);

int code = https.POST(json);

Serial.printf("ThingsBoard POST: %d\n", code);

https.end();

} else {

Serial.println("ThingsBoard connection failed.");

}

// === Send to Flask API ===

WiFiClient httpClient;

HTTPClient http;

if (http.begin(httpClient, flaskAPI)) {

http.addHeader("Content-Type", "application/json");

StaticJsonDocument<256> doc;

doc["kit_id"] = kitID;

doc["temperature"] = temp;

doc["humidity"] = hum;

doc["co"] = calibrated_CO;

doc["air_quality"] = ppm_air;

doc["pm25"] = calibrated_PM25;

doc["pm10"] = calibrated_PM10;

String json;

serializeJson(doc, json);

int code = http.POST(json);

Serial.printf("Flask POST: %d\n", code);

http.end();

} else {

Serial.println("Flask API connection failed.");

}

delay(5000);  // Update interval

}

TFT Output:

Air Quality Monitor

------------------------

Temp:      27.6 C

Humidity:  58.4 %

CO:        1.45 ppm

Air Q:     150.82 ppm

PM2.5:     16.3 ug

PM10:      52.8 ug