CAN-Based Environmental

Monitoring System

User Manual

| Filling Reference : | Integra/AMU/4.1/User Manual |  |
| --- | --- | --- |
| Document Title : | User Manual |  |
| Version : | Draft |  |
| Document Author: | Mr. Sabarish |  |
| Date Created : | 01/06/2025 |  |
| Date Modified : | 01/06/2025 |  |

|  | Reviewed By | Accepted By |
| --- | --- | --- |
| Signature & Date |  |  |
| Name | Mr. Bala |  |
| Designation | Technical Consultant |  |

Change History:

| Version | Date | Author | Change History |
| --- | --- | --- | --- |
| Draft | 01/06/2025 | Mr. Sabarish | Initial Draft Created |

Distribution List:

| Name | Email Address |
| --- | --- |
| Mr. Bala | bala@integratech.com.sg |
| Mr. Hari | hari@integratech.com.sg |
| Mr. Selva Pandian | selvapandian@integratech.com.sg |
| Mr. Sabarish | sabarish@integratech.com.sg |
| Mr. Sivaguru | sivaguru@integratech.com.sg |

Table of Contents

| Contents 1. Project Overview 4 2. Objectives 4 3. Hardware Component 5 4. Software Components 6 5. Circuit Diagram and Wiring 7 6. Code Explanation 10 7. Setup and Usage Instructions 11 8. Data Flow and Communication 12 9. Trouble shooting and FAQs 13 10. Code Examples 14 CLASS 1 - Random Integers, Send a 16-bit random integer (0–9999) over CAN, Transmitter Code (transmitter_random_integer) 14 CLASS 2 - Random Floats, Emulate a float like 23.5 by sending 235 (scaled by 10), Transmitter – transmitter_random_float. 16 CLASS 3 - Multiple Random Numbers (2 values), Transmitter – transmitter_multi_random 17 CLASS 4 - Random ASCII Bytes, Transmitter – transmitter_ascii 19 CLASS 5 - Timestamps (Fake RTC), Send HH:MM:SS using 3 bytes ,Transmitter – transmitter_timestamp 20 CLASS 6 - Status Flags / Booleans, Send bitmask for 8 flags (1 byte), Transmitter – transmitter_flags 22 CLASS 7 - Simulated Sensor Data, Fake temperature, humidity, gas (3 x 8-bit), Transmitter – transmitter_sim_sensor 23 CLASS 8 - Control Commands, Send command code: START (0x01), STOP (0x02), etc.,Transmitter – transmitter_command 25 CLASS 9 - Message ID Variants, Use different can_id values for different sensor types (e.g., temp = 0x401, hum = 0x402, gas = 0x403), Transmitter – transmitter_ids 26 CLASS10 - Byte Encoding Schemes (Pack 2 values in 1 byte), Pack 2 nibbles (4-bit each) into 1 byte, like voltage level (0–15) and battery (0–15), Transmitter – transmitter_packed_bytes 28 |  |
| --- | --- |

- Project Overview

This project presents a distributed environmental monitoring system utilizing the Controller Area Network (CAN) bus for inter-device communication. The system comprises two primary nodes:

- Transmitter Node: Equipped with a DHT11 sensor, this node measures temperature and humidity and transmits the data over the CAN bus.

- Receiver Node: This node receives the transmitted data, displays it on a TFT screen, and forwards it via Wi-Fi to two destinations:

  - ThingsBoard, an IoT dashboard for real-time visualization.

  - A Flask-based backend integrated with MongoDB for data storage and further analysis.

- Objectives

- Continuously monitor temperature and humidity in real time.

- Transmit sensor data using the CAN protocol for reliable inter-node communication.

- Display received data on a TFT screen for local visualization.

- Upload sensor readings to both ThingsBoard (IoT dashboard) and a custom Flask API connected to a MongoDB backend.

- Enable seamless OTA (Over-the-Air) updates and provide a user-friendly Wi-Fi configuration interface.

- Hardware Component

| Component | Quantity | Description |
| --- | --- | --- |
| ESP32 Dev Module | 2 | Microcontroller board with WiFi & SPI |
| MCP2515 CAN Module | 2 | CAN transceiver module with TJA1050 |
| DHT11 Sensor | 1 | Temperature and humidity sensor |
| TFT 2.2" ILI9225 | 1 | Color TFT display |
| Jumper wires | Many | For circuit connections |
| Breadboard (optional) | 2 | For prototyping |
| Power Supply | 2 | USB/5V power source for ESP32 |

- Software Components

- Development Environment:

  - Arduino IDE with ESP32 board support installed via the Board Manager.

- Libraries Used:

  - SPI.h – Enables SPI communication for interfacing with the TFT display.

  - mcp2515.h – Driver library for the MCP2515 CAN controller.

  - DHT.h – Used to interface with the DHT11 temperature and humidity sensor.

  - TFT_22_ILI9225.h – Driver for the 2.2” TFT display based on the ILI9225 controller.

  - WiFiManager.h – Simplifies Wi-Fi configuration using a captive portal.

  - ArduinoOTA.h – Enables Over-the-Air firmware updates.

  - HTTPClient.h – Facilitates HTTP requests for cloud communication.

  - ArduinoJson.h – Used for parsing and generating JSON data structures.

- Circuit Diagram and Wiring

Transmitter Node (ESP32 + DHT11 + MCP2515)

| DHT11 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| DATA | GPIO 22 |

| MCP2515 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| CS | GPIO 17 |
| SO (MISO) | GPIO 19 |
| SI (MOSI) | GPIO 23 |
| SCK | GPIO 18 |
| INT | Not used |

Receiver Node (ESP32 + TFT + MCP2515):

| TFT Pin | ESP32 Pin |
| --- | --- |
| CS | GPIO 15 |
| RS (DC) | GPIO 25 |
| RST | GPIO 26 |
| SDI (MOSI) | GPIO 13 |
| SCL (SCK) | GPIO 14 |
| LED | GPIO 0 |

| MCP2515 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| CS | GPIO 17 |
| SO (MISO) | GPIO 19 |
| SI (MOSI) | GPIO 23 |
| SCK | GPIO 18 |

Note: TFT uses HSPI, MCP2515 uses VSPI to avoid SPI conflicts.

- Code Explanation

Transmitter Code Highlights

- Reads temperature and humidity values from the DHT11 sensor.

- Encapsulates the sensor data into a CAN frame (canMsg) for transmission.

- Sends the CAN frame with identifier 0x101 at 2-second intervals for consistent data updates.

cpp

CopyEdit

canMsg.can_id  = 0x101;

canMsg.can_dlc = 8;

canMsg.data[0] = (int)temperature;

canMsg.data[1] = (int)humidity;

Receiver Code Highlights

- Monitors the CAN bus for incoming messages with identifier 0x101.

- Displays the received sensor data on a TFT screen for real-time local monitoring.

- Transmits the data securely to ThingsBoard using HTTPS for cloud-based visualization.

- Sends the same data to a custom Flask API over HTTP for backend storage and processing.

- Setup and Usage Instructions

Flashing the code

- Install ESP32 board support in the Arduino IDE via the Board Manager.

- Install all required libraries listed in the software requirements section.

- Connect the ESP32 boards to your computer using USB cables.

- Flash the transmitter and receiver code to respective boards

Wiring

- Carefully follow the wiring instructions provided in the documentation.

- Ensure all components are powered with the correct voltage - 3.3V for ESP32 and all connected peripherals.

Running the System

- Power on both ESP32 modules using a stable power source.

- The transmitter node will begin sending sensor data over the CAN bus.

- The receiver node will:

  - Display the received data on the TFT screen.

  - Upload the data to:

    - ThingsBoard (for real-time visualization)

    - A custom Flask API (for storage and processing)

- Data Flow and Communication

- The CAN bus transmits only raw integer values, specifically temperature and humidity readings.

- The receiver node parses these values, displays them on a TFT screen, and uploads the processed data to cloud services such as ThingsBoard and a custom Flask API.

- Trouble shooting and FAQs

Q1. MCP2515 not initializing?

- Ensure CS pin is correct.

- Ensure SPI pins are not conflicting.

- Ensure module is powered with 3.3V.

Q2. No data received on the receiver side?

- Check CAN wiring.

- Ensure both nodes are using same bitrate (500 Kbps).

- Verify that both MCP2515 modules are operational.

Q3. DHT11 returns NaN?

- Ensure sensor is properly connected.

- Add a pull-up resistor (~10KΩ) on the data pin.

Q4. TFT screen not displaying anything?

- Verify HSPI pins are correct.

- Confirm backlight (LED pin) is powered.

- Use Serial Monitor to debug if TFT initialization succeeded.

Q5. WiFi not connecting?

- ESP32 opens AP mode on first boot (CAN-EnvMonitor).

- Connect to it, and configure WiFi via captive portal.

- Code Examples

## CLASS 1 - Random Integers, Send a 16-bit random integer (0–9999) over CAN, Transmitter Code (transmitter_random_integer)

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);  // CS pin

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

if (mcp2515.reset() == MCP2515::ERROR_OK) {

Serial.println("MCP2515 Initialized");

} else {

Serial.println("MCP2515 Init Failed");

while (1);

}

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

randomSeed(analogRead(0));

Serial.println("Ready to send random integers over CAN...");

}

void loop() {

int randomNumber = random(0, 10000);  // 0–9999

Serial.print("Sending number: ");

Serial.println(randomNumber);

canMsg.can_id  = 0x301;

canMsg.can_dlc = 2;

canMsg.data[0] = (randomNumber >> 8) & 0xFF;

canMsg.data[1] = randomNumber & 0xFF;

mcp2515.sendMessage(&canMsg);

delay(2000);

}

Receiver Code (receiver_random_integer)

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);  // CS pin

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

if (mcp2515.reset() == MCP2515::ERROR_OK) {

Serial.println("MCP2515 Initialized");

} else {

Serial.println("MCP2515 Init Failed");

while (1);

}

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Ready to receive random integers...");

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK) {

if (canMsg.can_id == 0x301 && canMsg.can_dlc == 2) {

int number = (canMsg.data[0] << 8) | canMsg.data[1];

Serial.print("Received number: ");

Serial.println(number);

}

}

delay(100);

}

CAN ID: 0x300
Transmitted: Random 16-bit integer (0–9999)

Output:

Received number: 7834

Received number: 1234

Received number: 9999

## CLASS 2 - Random Floats, Emulate a float like 23.5 by sending 235 (scaled by 10), Transmitter – transmitter_random_float.

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

randomSeed(analogRead(0));

Serial.println("Sending scaled float values...");

}

void loop() {

float value = random(200, 350) / 10.0;  // e.g., 20.0 to 35.0

int scaled = value * 10;

Serial.print("Sending float: ");

Serial.println(value);

canMsg.can_id = 0x302;

canMsg.can_dlc = 2;

canMsg.data[0] = (scaled >> 8) & 0xFF;

canMsg.data[1] = scaled & 0xFF;

mcp2515.sendMessage(&canMsg);

delay(2000);

}

Receiver – receiver_random_float

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving scaled float values...");

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK && canMsg.can_id == 0x302) {

int scaled = (canMsg.data[0] << 8) | canMsg.data[1];

float value = scaled / 10.0;

Serial.print("Received float: ");

Serial.println(value);

}

}

CAN ID: 0x302
Transmitted: 235 → 23.5°C

Output:

Received float: 23.5

Received float: 28.4

Received float: 19.0

## CLASS 3 - Multiple Random Numbers (2 values), Transmitter – transmitter_multi_random

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

randomSeed(analogRead(0));

Serial.println("Sending 2 random integers...");

}

void loop() {

int a = random(0, 500);

int b = random(0, 500);

Serial.print("A = "); Serial.print(a);

Serial.print(", B = "); Serial.println(b);

canMsg.can_id = 0x303;

canMsg.can_dlc = 4;

canMsg.data[0] = a >> 8;

canMsg.data[1] = a & 0xFF;

canMsg.data[2] = b >> 8;

canMsg.data[3] = b & 0xFF;

mcp2515.sendMessage(&canMsg);

delay(2000);

}

Receiver – receiver_multi_random

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving 2 random integers...");

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK && canMsg.can_id == 0x303) {

int a = (canMsg.data[0] << 8) | canMsg.data[1];

int b = (canMsg.data[2] << 8) | canMsg.data[3];

Serial.print("Received A: "); Serial.print(a);

Serial.print(", B: "); Serial.println(b);

}

}

CAN ID: 0x303
Transmitted: Two 16-bit numbers

Output:

Received A: 120, B: 451

Received A: 3200, B: 995

Received A: 0, B: 10000

## CLASS 4 - Random ASCII Bytes, Transmitter – transmitter_ascii

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

const char charset[] = "ABCDEFGHIJKLMNOPQRSTUVWXYZ";

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

randomSeed(analogRead(0));

}

void loop() {

canMsg.can_id = 0x304;

canMsg.can_dlc = 8;

for (int i = 0; i < 8; i++) {

canMsg.data[i] = charset[random(0, 26)];

}

Serial.print("Sending: ");

for (int i = 0; i < 8; i++) Serial.print((char)canMsg.data[i]);

Serial.println();

mcp2515.sendMessage(&canMsg);

delay(2000);

}

Receiver – receiver_ascii

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving ASCII string...");

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK && canMsg.can_id == 0x304) {

Serial.print("Received: ");

for (int i = 0; i < canMsg.can_dlc; i++) {

Serial.print((char)canMsg.data[i]);

}

Serial.println();

}

}

CAN ID: 0x304
Transmitted: 8 random ASCII bytes

Output:

Received text: W9HGLR2M

Received text: AQZWSXED

Received text: 7YTBKLPV

## CLASS 5 - Timestamps (Fake RTC), Send HH:MM:SS using 3 bytes ,Transmitter – transmitter_timestamp

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Sending fake timestamps...");

}

void loop() {

byte hour = random(0, 24);

byte minute = random(0, 60);

byte second = random(0, 60);

Serial.printf("Sending time: %02d:%02d:%02d\n", hour, minute, second);

canMsg.can_id = 0x305;

canMsg.can_dlc = 3;

canMsg.data[0] = hour;

canMsg.data[1] = minute;

canMsg.data[2] = second;

mcp2515.sendMessage(&canMsg);

delay(1000);

}

Receiver – receiver_timestamp

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving timestamp...");

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK && canMsg.can_id == 0x305) {

byte hour = canMsg.data[0];

byte minute = canMsg.data[1];

byte second = canMsg.data[2];

Serial.printf("Received time: %02d:%02d:%02d\n", hour, minute, second);

}

}

CAN ID: 0x305
Transmitted: 3 bytes → hour, min, sec

Output:

Received time: 14:36:08

Received time: 07:15:59

Received time: 23:59:59

## CLASS 6   - Status Flags / Booleans, Send bitmask for 8 flags (1 byte), Transmitter – transmitter_flags

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

randomSeed(analogRead(0));

Serial.println("Sending status flags...");

}

void loop() {

byte status = random(0, 256); // 0b00000000 to 0b11111111

Serial.print("Sending status flags: ");

Serial.println(status, BIN);

canMsg.can_id = 0x306;

canMsg.can_dlc = 1;

canMsg.data[0] = status;

mcp2515.sendMessage(&canMsg);

delay(1500);

}

Receiver – receiver_flags

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving flags...");

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK && canMsg.can_id == 0x306) {

byte flags = canMsg.data[0];

Serial.print("Flags: ");

Serial.println(flags, BIN);

}

}

CAN ID: 0x306
Transmitted: 1 byte = 8 status flags

Output:

Flags: 10101010

Flags: 11001100

Flags: 00011110

## CLASS 7 - Simulated Sensor Data, Fake temperature, humidity, gas (3 x 8-bit), Transmitter – transmitter_sim_sensor

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Sending fake sensor data...");

}

void loop() {

byte temp = random(20, 35);  // °C

byte hum  = random(30, 60);  // %

byte gas  = random(100, 200); // ppm

Serial.printf("Temp: %d, Hum: %d, Gas: %d\n", temp, hum, gas);

canMsg.can_id = 0x307;

canMsg.can_dlc = 3;

canMsg.data[0] = temp;

canMsg.data[1] = hum;

canMsg.data[2] = gas;

mcp2515.sendMessage(&canMsg);

delay(2000);

}

Receiver – receiver_sim_sensor

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving sensor values...");

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK && canMsg.can_id == 0x307) {

byte temp = canMsg.data[0];

byte hum  = canMsg.data[1];

byte gas  = canMsg.data[2];

Serial.printf("Temp: %d°C, Hum: %d%%, Gas: %dppm\n", temp, hum, gas);

}

}

CAN ID: 0x307
Transmitted: Temperature, Humidity, Gas

Output:

Temp: 28°C, Hum: 48%, Gas: 150ppm

Temp: 25°C, Hum: 60%, Gas: 190ppm

Temp: 30°C, Hum: 35%, Gas: 110ppm

## CLASS 8 - Control Commands, Send command code: START (0x01), STOP (0x02), etc.,Transmitter – transmitter_command

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

byte commands[] = {0x01, 0x02, 0x03}; // START, STOP, RESET

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Sending control commands...");

}

void loop() {

byte cmd = commands[random(0, 3)];

canMsg.can_id = 0x308;

canMsg.can_dlc = 1;

canMsg.data[0] = cmd;

Serial.print("Sending command: ");

Serial.println(cmd, HEX);

mcp2515.sendMessage(&canMsg);

delay(2000);

}

Receiver – receiver_command

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame canMsg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving command...");

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK && canMsg.can_id == 0x308) {

byte cmd = canMsg.data[0];

Serial.print("Command received: ");

switch (cmd) {

case 0x01: Serial.println("START"); break;

case 0x02: Serial.println("STOP"); break;

case 0x03: Serial.println("RESET"); break;

default: Serial.println("UNKNOWN");

}

}

}

CAN ID: 0x308
Transmitted: 1 byte = command code

Output:

Command received: START

Command received: STOP

Command received: RESET

Command received: UNKNOWN

## CLASS 9 - Message ID Variants, Use different can_id values for different sensor types (e.g., temp = 0x401, hum = 0x402, gas = 0x403), Transmitter – transmitter_ids

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame msg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Sending using multiple CAN IDs...");

}

void loop() {

msg.can_dlc = 1;

msg.can_id = 0x401;

msg.data[0] = random(20, 35);

mcp2515.sendMessage(&msg);

Serial.print("Sent TEMP: ");

Serial.println(msg.data[0]);

delay(500);

msg.can_id = 0x402;

msg.data[0] = random(30, 60);

mcp2515.sendMessage(&msg);

Serial.print("Sent HUM: ");

Serial.println(msg.data[0]);

delay(500);

msg.can_id = 0x403;

msg.data[0] = random(100, 200);

mcp2515.sendMessage(&msg);

Serial.print("Sent GAS: ");

Serial.println(msg.data[0]);

delay(1500);

}

Receiver – receiver_ids

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame msg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving data from different CAN IDs...");

}

void loop() {

if (mcp2515.readMessage(&msg) == MCP2515::ERROR_OK) {

switch (msg.can_id) {

case 0x401:

Serial.print("TEMP Received: ");

Serial.println(msg.data[0]);

break;

case 0x402:

Serial.print("HUMIDITY Received: ");

Serial.println(msg.data[0]);

break;

case 0x403:

Serial.print("GAS Received: ");

Serial.println(msg.data[0]);

break;

}

}

}

CAN IDs: 0x401, 0x402, 0x403
Transmitted: Temp, Humidity, Gas — each via separate ID

Output:

TEMP Received: 28

HUMIDITY Received: 50

GAS Received: 180

TEMP Received: 31

HUMIDITY Received: 45

GAS Received: 130

## CLASS10 - Byte Encoding Schemes (Pack 2 values in 1 byte), Pack 2 nibbles (4-bit each) into 1 byte, like voltage level (0–15) and battery (0–15), Transmitter – transmitter_packed_bytes

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame msg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Sending packed byte values...");

}

void loop() {

byte voltage = random(0, 16);  // 4-bit

byte battery = random(0, 16);  // 4-bit

byte packed = (voltage << 4) | (battery & 0x0F);

msg.can_id = 0x40A;

msg.can_dlc = 1;

msg.data[0] = packed;

mcp2515.sendMessage(&msg);

Serial.print("Sent voltage: ");

Serial.print(voltage);

Serial.print(", battery: ");

Serial.println(battery);

delay(2000);

}

Receiver – receiver_packed_bytes

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(17);

struct can_frame msg;

void setup() {

Serial.begin(115200);

SPI.begin();

mcp2515.reset();

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("Receiving packed byte values...");

}

void loop() {

if (mcp2515.readMessage(&msg) == MCP2515::ERROR_OK && msg.can_id == 0x40A) {

byte packed = msg.data[0];

byte voltage = (packed >> 4) & 0x0F;

byte battery = packed & 0x0F;

Serial.print("Voltage: ");

Serial.print(voltage);

Serial.print(", Battery: ");

Serial.println(battery);

}

}

CAN ID: 0x40A
Transmitted: Voltage & Battery Level packed into 1 byte

Output:

Voltage: 12, Battery: 7

Voltage: 3, Battery: 14

Voltage: 15, Battery: 0