EV Monitoring System

User Manual

| Filling Reference : | Integra/AMU/KIT4.2/User Manual |  |
| --- | --- | --- |
| Document Title : | User Manual |  |
| Version : | Draft |  |
| Document Author: | Mr. Sabarish |  |
| Date Created : | 01/06/2025 |  |
| Date Modified : | 01/06/2025 |  |

|  | Reviewed By | Accepted By |
| --- | --- | --- |
| Signature & Date |  |  |
| Name | Mr. Bala |  |
| Designation | Technical Consultant |  |

Change History:

| Version | Date | Author | Change History |
| --- | --- | --- | --- |
| Draft | 01/06/2025 | Mr. Sabarish | Initial Draft Created |

Distribution List:

| Name | Email Address |
| --- | --- |
| Mr. Bala | bala@integratech.com.sg |
| Mr. Hari | hari@integratech.com.sg |
| Mr. Selva Pandian | selvapandian@integratech.com.sg |
| Mr. Sabarish | sabarish@integratech.com.sg |
| Mr. Sivaguru | sivaguru@integratech.com.sg |

Table of Contents

| Contents 1. Project Overview 4 2. Objectives 6 3. Hardware Component 7 4. Software Components 8 5. Circuit Diagram and Wiring 9 6. Code Explanation 11 7. Setup and Usage Instructions 13 8. Data Flow and Communication 14 9. Trouble shooting and FAQs 15 10. Code Examples 16 CLASS 1 - INA3221 (I) only 16 CLASS 2 - NTC (N) only 17 CLASS 3 - TFT (T) only 17 CLASS 4 - INA3221 + NTC (I+N) 18 CLASS 5 - INA3221 + TFT (I+T) 19 CLASS 6 - NTC + TFT (N+T) 20 CLASS 7 - INA3221 + NTC + TFT (I+N+T) 21 CLASS 8 - CAN bus only (C), CAN transmitter & receiver, Transmitter (send dummy data): 22 CLASS 9 - INA3221 + CAN (I+C), Transmitter: 23 CLASS 10 - NTC + CAN (N+C) 26 CLASS 11 - TFT + CAN (T+C) 28 CLASS 12 - INA3221 + NTC + CAN (I+N+C) 30 CLASS 13 - INA3221 + TFT + CAN (I+T+C) 33 CLASS 14 - NTC + TFT + CAN (N+T+C) 35 CLASS 15 -MAIN CODE(All Sensor Combination With Logic): 37 |  |
| --- | --- |

- Project Overview

The Internet-Connected Electric Vehicle (EV) Monitoring and Control System introduces a smart, Internet-connected system for monitoring and controlling Electric Vehicles (EVs), built using two ESP32 microcontroller boards:

- Transmitter Unit: Installed within the EV, this unit is responsible for:

  - Collecting real-time data from various sensors

  - Controlling relays for safety and charging operations

  - Transmitting data via the CAN Bus protocol

- Receiver Unit: Positioned externally, this unit:

  - Receives data from the CAN Bus

  - Displays key metrics on a TFT screen

  - Sends telemetry data to both a local Flask-based server and the ThingsBoard IoT platform

Key Features and Capabilities:

- Real-Time Monitoring of:

  - Battery voltage and current

  - Charging voltage and current

  - Motor voltage, current, and temperature

- Automated Relay Control for safety mechanisms and charging logic

- Data Logging to both local storage and cloud-based servers

- Wireless OTA (Over-the-Air) Firmware Updates for seamless maintenance

- User-Friendly Wi-Fi Configuration using WiFiManager

- Objectives

The primary objective is to safely monitor and control the electrical system of an Electric Vehicle (EV), ensuring that system data is both locally displayed and remotely accessible for diagnostics and analytics.

Specific Objectives:

- Transmit Sensor Data between ESP32 microcontrollers using the CAN Bus protocol

- Automate Control of the motor and charger based on predefined safety thresholds

- Display System Metrics on a graphical TFT screen for real-time visualization

- Log Data to:

  - A local server via a Flask-based REST API

  - A remote IoT dashboard using the ThingsBoard platform

- Enable OTA (Over-the-Air) Firmware Updates via Wi-Fi for remote maintenance

- Support Wi-Fi Provisioning using WiFiManager for easy network configuration

- Hardware Component

| Component | Specs/Notes | Quantity |
| --- | --- | --- |
| ESP32 Dev Kit (DOIT) | Dual-core WiFi/BLE microcontroller | 2 |
| INA3221 Module | Triple-channel voltage/current sensor | 1 |
| MCP2515 CAN Module | CAN Controller with TJA1050 Transceiver | 2 |
| Thermistor (NTC 10k) | Used for motor temperature sensing | 1 |
| TFT Display (ILI9225) | 2.2" 176x220 SPI-based TFT | 1 |
| Relay Module | 2-channel, 5V or 3.3V logic compatible | 1 |
| 0.1Ω Shunt Resistor | Current sensing with INA3221 | 3 |
| Power Supply | 12V DC or USB | 1 |
| Cables, Breadboard | For testing and prototyping | — |

- Software Components

IDE & Board:

Arduino IDE with ESP32 Board Support

Libraries:

| Library | Purpose |
| --- | --- |
| INA3221 | Read multi-channel voltages/currents |
| WiFiManager | WiFi configuration via browser |
| ArduinoOTA | OTA firmware upload |
| mcp2515 | CAN bus communication |
| SPI, Wire | SPI and I2C communication |
| TFT_22_ILI9225 | TFT display rendering |
| ArduinoJson | Create and parse JSON payloads |
| HTTPClient | POST data to REST APIs |

- Circuit Diagram and Wiring

Transmitter Wiring Overview

| ESP32 Pin | Component | Description |
| --- | --- | --- |
| GPIO 16 | INA3221 SDA | I2C Data |
| GPIO 17 | INA3221 SCL | I2C Clock |
| GPIO 34 | Thermistor | Analog input (Voltage Divider) |
| GPIO 21 | Motor Relay | Relay control |
| GPIO 22 | Charger Relay | Relay control |
| GPIO 5 | MCP2515 CS | SPI Chip Select |
| GPIO 23 | MCP2515 MOSI | SPI Data |
| GPIO 19 | MCP2515 MISO | SPI Data |
| GPIO 18 | MCP2515 CLK | SPI Clock |

Receiver Wiring Overview:

| ESP32 Pin | Component | Description |
| --- | --- | --- |
| GPIO 13 | TFT SDI (MOSI) | SPI Data |
| GPIO 14 | TFT CLK (SCK) | SPI Clock |
| GPIO 15 | TFT CS | Chip Select |
| GPIO 25 | TFT RS | Register Select |
| GPIO 26 | TFT RST | Reset |
| GPIO 0 | TFT LED | Backlight Control |
| GPIO 5 | MCP2515 CS | SPI Chip Select |

CAN Bus Wiring:

- Connect CAN_H ↔ CAN_H

- Connect CAN_L ↔ CAN_L

- Place 120Ω termination resistors at both ends of the bus.

- Code Explanation

Transmitter Code Breakdown

- INA3221 Sensor: Measures voltage and current from the charger, battery, and motor.

- Thermistor: Connected to an ADC channel; calculates motor temperature using the Steinhart-Hart equation.

- Relay Control Logic:

  - Battery full → Stop charging

  - Battery low → Start charging

  - Motor overheating → Stop motor

- CAN Bus Transmission:

  - Frame 0x101: Motor temperature, battery voltage, battery current

  - Frame 0x102: Charger voltage/current, motor voltage

  - Frame 0x103: Motor current, relay states

- OTA & Wi-Fi:

  - OTA firmware updates enabled

  - Wi-Fi auto-configuration via captive portal using WiFiManager

Receiver Code Breakdown

- CAN Bus Reception:

  - Receives and parses all three CAN frames

  - Updates internal variables accordingly

- TFT Display:

  - Graphically displays all system parameters with clear labels and units

- API Communication:

  - Sends structured JSON data to:

    - Local Flask server

    - Remote ThingsBoard IoT dashboard

- Error Handling:

  - Uses NaN for invalid or missing sensor data

  - Implements conditional rendering logic to avoid displaying erroneous values

- Setup and Usage Instructions

First Time Setup

- Connect the hardware according to the wiring diagram.

- Upload the transmitter code to ESP32 #1.

- Upload the receiver code to ESP32 #2.

- Open the Arduino Serial Monitor at 115200 baud for debugging.

Wi-Fi Configuration

- When powered on, each ESP32 will create its own Wi-Fi Access Point (AP).

- Connect your phone to the following networks:

- KIT4.2-Transmitter

- KIT4.2-Receiver

- A captive portal will automatically open.

- Select your Wi-Fi network and enter the credentials.

OTA (Over-the-Air) Update

- Ensure both ESP32 devices are connected to the same Wi-Fi network.

- In the Arduino IDE, select the appropriate OTA port (e.g., EV-System_transmitter).

- Upload the sketch as you would normally.

- Data Flow and Communication

| Frame ID | Data |
| --- | --- |
| 0x101 | Temp (×10), Battery V (×100), I (mA) |
| 0x102 | Charger V/I, Motor V (scaled) |
| 0x103 | Motor I (mA), Relay bits |

- Trouble shooting and FAQs

CAN Bus Errors

- Verify SPI wiring: Check connections for CS, MISO, MOSI, and SCK.

- Ensure both CAN modules are set to the same bitrate.

- Use setBitrate(CAN_500KBPS, MCP_8MHZ) on both transmitter and receiver.

- Confirm that 120Ω termination resistors are installed at both ends of the CAN bus.

WiFiManager Not Appearing

- Reboot the ESP32.

- Clear saved credentials by holding the BOOT button during startup.

- Make sure the ESP32 is not auto-connecting to a previously saved Wi-Fi network.

OTA Port Not Showing in Arduino IDE?

- Confirm the ESP32 is on the same subnet as your computer.

- Ensure mDNS is functioning (try disabling VPN or firewall temporarily).

- Restart the Arduino IDE or manually rescan for network ports.

Invalid Sensor Readings?

- Double-check configuration values:

- VREF

- SERIES_RESISTOR

- B_COEFFICIENT

- Calibrate the ADC using esp_adc_cal_characterize().

- Ensure the thermistor wiring is correct (not reversed or floating).

- Code Examples

## CLASS 1 - INA3221 (I) only

#include <Wire.h>

#include <INA3221.h>

INA3221 ina3221;

void setup() {

Serial.begin(115200);

Wire.begin();

ina3221.begin();

}

void loop() {

float voltage = ina3221.getBusVoltage_V(1);

float current = ina3221.getCurrent_mA(1) / 1000.0;

Serial.printf("Voltage: %.2f V, Current: %.2f A\n", voltage, current);

delay(1000);

}

Serial Output:

Voltage: 12.34

Current: 1.23

Voltage: 12.31

Current: 1.22

## CLASS 2 - NTC (N) only

#define NTC_PIN A0

void setup() {

Serial.begin(115200);

}

float readNTCTemperature() {

int raw = analogRead(NTC_PIN);

float resistance = (1023.0 / raw - 1.0) * 10000;

float tempK = 1.0 / (0.001129148 + 0.000234125 * log(resistance) + 0.0000000876741 * pow(log(resistance), 3));

return tempK - 273.15;

}

void loop() {

float temp = readNTCTemperature();

Serial.printf("Temp: %.2f C\n", temp);

delay(1000);

}

Serial Output:

Motor Temp: 35.67 C

Motor Temp: 35.65 C

## CLASS 3 - TFT (T) only

#include <TFT_eSPI.h>

TFT_eSPI tft = TFT_eSPI();

void setup() {

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(3);

tft.drawString("TFT Only", 10, 40);

}

void loop() {

// no update

}

Serial Output: (none)

TFT Display:

Hello World

## CLASS 4 - INA3221 + NTC (I+N)

#include <Wire.h>

#include <INA3221.h>

#define NTC_PIN A0

INA3221 ina3221;

float readNTCTemperature() {

int raw = analogRead(NTC_PIN);

float resistance = (1023.0 / raw - 1.0) * 10000;

float tempK = 1.0 / (0.001129148 + 0.000234125 * log(resistance) + 0.0000000876741 * pow(log(resistance), 3));

return tempK - 273.15;

}

void setup() {

Serial.begin(115200);

Wire.begin();

ina3221.begin();

}

void loop() {

float voltage = ina3221.getBusVoltage_V(1);

float current = ina3221.getCurrent_mA(1) / 1000.0;

float temp = readNTCTemperature();

Serial.printf("Voltage: %.2f V, Current: %.2f A, Temp: %.2f C\n", voltage, current, temp);

delay(1000);

}

Serial Output:

Voltage: 12.34

Current: 1.23

Motor Temp: 35.67 C

Voltage: 12.31

Current: 1.22

Motor Temp: 35.65 C

...

## CLASS 5 - INA3221 + TFT (I+T)

#include <Wire.h>

#include <INA3221.h>

#include <TFT_eSPI.h>

INA3221 ina3221;

TFT_eSPI tft = TFT_eSPI();

void setup() {

Serial.begin(115200);

Wire.begin();

ina3221.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

}

void loop() {

float voltage = ina3221.getBusVoltage_V(1);

float current = ina3221.getCurrent_mA(1) / 1000.0;

tft.fillScreen(TFT_BLACK);

tft.drawString("Voltage: " + String(voltage, 2) + " V", 10, 20);

tft.drawString("Current: " + String(current, 2) + " A", 10, 50);

delay(1000);

}

Serial Output:

Voltage: 12.34

Current: 1.23

Voltage: 12.31

Current: 1.22

...

TFT Display:

Voltage: 12.34 V

Current: 1.23 A

## CLASS 6 - NTC + TFT (N+T)

#include <TFT_eSPI.h>

#define NTC_PIN A0

TFT_eSPI tft = TFT_eSPI();

float readNTCTemperature() {

int raw = analogRead(NTC_PIN);

float resistance = (1023.0 / raw - 1.0) * 10000;

float tempK = 1.0 / (0.001129148 + 0.000234125 * log(resistance) + 0.0000000876741 * pow(log(resistance), 3));

return tempK - 273.15;

}

void setup() {

Serial.begin(115200);

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

}

void loop() {

float temp = readNTCTemperature();

tft.fillScreen(TFT_BLACK);

tft.drawString("Temp: " + String(temp, 2) + " C", 10, 20);

delay(1000);

}

Serial Output:

Motor Temp: 35.67 C

Motor Temp: 35.65 C

## CLASS 7 - INA3221 + NTC + TFT (I+N+T)

#include <Wire.h>

#include <INA3221.h>

#include <TFT_eSPI.h>

#define NTC_PIN A0

INA3221 ina3221;

TFT_eSPI tft = TFT_eSPI();

float readNTCTemperature() {

int raw = analogRead(NTC_PIN);

float resistance = (1023.0 / raw - 1.0) * 10000;

float tempK = 1.0 / (0.001129148 + 0.000234125 * log(resistance) + 0.0000000876741 * pow(log(resistance), 3));

return tempK - 273.15;

}

void setup() {

Serial.begin(115200);

Wire.begin();

ina3221.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

}

void loop() {

float voltage = ina3221.getBusVoltage_V(1);

float current = ina3221.getCurrent_mA(1) / 1000.0;

float temp = readNTCTemperature();

tft.fillScreen(TFT_BLACK);

tft.drawString("Voltage: " + String(voltage, 2) + " V", 10, 20);

tft.drawString("Current: " + String(current, 2) + " A", 10, 50);

tft.drawString("Temp: " + String(temp, 2) + " C", 10, 80);

delay(1000);

}

Serial Output:

Voltage: 12.34

Current: 1.23

Motor Temp: 35.67 C

...

TFT Display:

Voltage: 12.34 V

Current: 1.23 A

Temp: 35.67 C

## CLASS 8 - CAN bus only (C), CAN transmitter & receiver, Transmitter (send dummy data):

#include <SPI.h>

#include <mcp_can.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

void setup() {

Serial.begin(115200);

SPI.begin();

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

byte data[8] = {1,2,3,4,5,6,7,8};

CAN.sendMsgBuf(0x200, 0, 8, data);

Serial.println("Sent CAN dummy data");

delay(1000);

}

Receiver:

#include <SPI.h>

#include <mcp_can.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

void setup() {

Serial.begin(115200);

SPI.begin();

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

if (CAN.checkReceive() == CAN_MSGAVAIL) {

unsigned long rxId;

byte len;

byte buf[8];

if (CAN.readMsgBuf(&rxId, &len, buf) == CAN_OK) {

Serial.print("Received CAN: ");

for (int i=0; i<len; i++) Serial.print(buf[i], HEX);

Serial.println();

}

}

}

Transmitter serial output:

Sent CAN dummy data

Sent CAN dummy data

Sent CAN dummy data

...

Receiver serial output:

Received CAN: 01 02 03 04 05 06 07 08

Received CAN: 01 02 03 04 05 06 07 08

...

## CLASS 9 - INA3221 + CAN (I+C), Transmitter:

#include <Wire.h>

#include <INA3221.h>

#include <SPI.h>

#include <mcp_can.h>

#define CAN_CS 5

#define CAN_INT 4

INA3221 ina3221;

MCP_CAN CAN(CAN_CS);

void setup() {

Serial.begin(115200);

Wire.begin();

SPI.begin();

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

ina3221.begin();

}

void loop() {

float voltage = ina3221.getBusVoltage_V(1);

float current = ina3221.getCurrent_mA(1) / 1000.0;

byte data[8];

memcpy(&data[0], &voltage, 4);

memcpy(&data[4], &current, 4);

CAN.sendMsgBuf(0x100, 0, 8, data);

Serial.printf("Sent Voltage: %.2f V, Current: %.2f A\n", voltage, current);

delay(1000);

}

Receiver:

#include <SPI.h>

#include <mcp_can.h>

#include <TFT_eSPI.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

TFT_eSPI tft = TFT_eSPI();

void setup() {

Serial.begin(115200);

SPI.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

if (CAN.checkReceive() == CAN_MSGAVAIL) {

unsigned long rxId;

byte len;

byte buf[8];

if (CAN.readMsgBuf(&rxId, &len, buf) == CAN_OK && rxId == 0x100) {

float voltage, current;

memcpy(&voltage, &buf[0], 4);

memcpy(&current, &buf[4], 4);

tft.fillScreen(TFT_BLACK);

tft.drawString("Voltage: " + String(voltage, 2) + " V", 10, 20);

tft.drawString("Current: " + String(current, 2) + " A", 10, 50);

Serial.printf("Received Voltage: %.2f V, Current: %.2f A\n", voltage, current);

}

}

}

Transmitter serial output:

Sent Voltage: 12.34 V, Current: 0.56 A

Sent Voltage: 12.31 V, Current: 0.57 A

...

Receiver serial output:

.Received Voltage: 12.34 V, Current: 0.56 A

Received Voltage: 12.31 V, Current: 0.57 A

...

Receiver TFT display:

Voltage: 12.34 V

Current: 0.56 A

## CLASS 10 - NTC + CAN (N+C)

Transmitter:

#include <SPI.h>

#include <mcp_can.h>

#define CAN_CS 5

#define CAN_INT 4

#define NTC_PIN A0

MCP_CAN CAN(CAN_CS);

float readNTCTemperature() {

int raw = analogRead(NTC_PIN);

float resistance = (1023.0 / raw - 1.0) * 10000;

float tempK = 1.0 / (0.001129148 + 0.000234125 * log(resistance) + 0.0000000876741 * pow(log(resistance), 3));

return tempK - 273.15;

}

void setup() {

Serial.begin(115200);

SPI.begin();

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

float temp = readNTCTemperature();

byte data[4];

memcpy(data, &temp, 4);

CAN.sendMsgBuf(0x101, 0, 4, data);

Serial.printf("Sent Temp: %.2f C\n", temp);

delay(1000);

}

Receiver:

#include <SPI.h>

#include <mcp_can.h>

#include <TFT_eSPI.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

TFT_eSPI tft = TFT_eSPI();

void setup() {

Serial.begin(115200);

SPI.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

if (CAN.checkReceive() == CAN_MSGAVAIL) {

unsigned long rxId;

byte len;

byte buf[8];

if (CAN.readMsgBuf(&rxId, &len, buf) == CAN_OK && rxId == 0x101) {

float temp;

memcpy(&temp, &buf[0], 4);

tft.fillScreen(TFT_BLACK);

tft.drawString("Temp: " + String(temp, 2) + " C", 10, 20);

Serial.printf("Received Temp: %.2f C\n", temp);

}

}

}

Transmitter serial output:

Sent Temp: 25.62 C

Sent Temp: 25.65 C

...

Receiver serial output:

Received Temp: 25.62 C

Received Temp: 25.65 C

...

Receiver TFT display:

Temp: 25.62 C

## CLASS 11 - TFT + CAN (T+C)

Transmitter:

// Just send dummy CAN data, no sensors

#include <SPI.h>

#include <mcp_can.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

void setup() {

Serial.begin(115200);

SPI.begin();

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

byte data[8] = {0xAA,0xBB,0xCC,0xDD,0x11,0x22,0x33,0x44};

CAN.sendMsgBuf(0x102, 0, 8, data);

Serial.println("Sent CAN dummy data");

delay(1000);

}

Receiver:

#include <SPI.h>

#include <mcp_can.h>

#include <TFT_eSPI.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

TFT_eSPI tft = TFT_eSPI();

void setup() {

Serial.begin(115200);

SPI.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

if (CAN.checkReceive() == CAN_MSGAVAIL) {

unsigned long rxId;

byte len;

byte buf[8];

if (CAN.readMsgBuf(&rxId, &len, buf) == CAN_OK && rxId == 0x102) {

tft.fillScreen(TFT_BLACK);

tft.drawString("CAN Msg:", 10, 20);

String s = "";

for (int i=0; i<len; i++) {

s += String(buf[i], HEX) + " ";

}

tft.drawString(s, 10, 50);

Serial.print("Received CAN: ");

for (int i=0; i<len; i++) Serial.print(buf[i], HEX); Serial.println();

}

}

}

Transmitter serial output:

Sent CAN dummy data

Sent CAN dummy data

...

Receiver serial output:

Received CAN: aa bb cc dd 11 22 33 44

Received CAN: aa bb cc dd 11 22 33 44

...

Receiver TFT display:

CAN Msg:

AA BB CC DD 11 22 33 44

## CLASS 12 - INA3221 + NTC + CAN (I+N+C)

Transmitter:

#include <Wire.h>

#include <INA3221.h>

#include <SPI.h>

#include <mcp_can.h>

#define CAN_CS 5

#define CAN_INT 4

#define NTC_PIN A0

INA3221 ina3221;

MCP_CAN CAN(CAN_CS);

float readNTCTemperature() {

int raw = analogRead(NTC_PIN);

float resistance = (1023.0 / raw - 1.0) * 10000;

float tempK = 1.0 / (0.001129148 + 0.000234125 * log(resistance) + 0.0000000876741 * pow(log(resistance), 3));

return tempK - 273.15;

}

void setup() {

Serial.begin(115200);

Wire.begin();

SPI.begin();

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

ina3221.begin();

}

void loop() {

float voltage = ina3221.getBusVoltage_V(1);

float current = ina3221.getCurrent_mA(1) / 1000.0;

float temp = readNTCTemperature();

byte data[12];

memcpy(&data[0], &voltage, 4);

memcpy(&data[4], &current, 4);

memcpy(&data[8], &temp, 4);

CAN.sendMsgBuf(0x103, 0, 12, data);

Serial.printf("Sent V: %.2f, I: %.2f, T: %.2f\n", voltage, current, temp);

delay(1000);

}

Receiver:

#include <SPI.h>

#include <mcp_can.h>

#include <TFT_eSPI.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

TFT_eSPI tft = TFT_eSPI();

void setup() {

Serial.begin(115200);

SPI.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

if (CAN.checkReceive() == CAN_MSGAVAIL) {

unsigned long rxId;

byte len;

byte buf[12];

if (CAN.readMsgBuf(&rxId, &len, buf) == CAN_OK && rxId == 0x103 && len == 12) {

float voltage, current, temp;

memcpy(&voltage, &buf[0], 4);

memcpy(&current, &buf[4], 4);

memcpy(&temp, &buf[8], 4);

tft.fillScreen(TFT_BLACK);

tft.drawString("Voltage: " + String(voltage, 2) + " V", 10, 20);

tft.drawString("Current: " + String(current, 2) + " A", 10, 50);

tft.drawString("Temp: " + String(temp, 2) + " C", 10, 80);

Serial.printf("Received V: %.2f, I: %.2f, T: %.2f\n", voltage, current, temp);

}

}

}

Transmitter serial output:

Sent V: 12.34, I: 0.56, T: 25.62

Sent V: 12.30, I: 0.55, T: 25.65

...

Receiver serial output:

Received V: 12.34, I: 0.56, T: 25.62

Received V: 12.30, I: 0.55, T: 25.65

...

Receiver TFT display:

Voltage: 12.34 V

Current: 0.56 A

Temp: 25.62 C

## CLASS 13 - INA3221 + TFT + CAN (I+T+C)

Transmitter:

#include <Wire.h>

#include <INA3221.h>

#include <SPI.h>

#include <mcp_can.h>

#define CAN_CS 5

#define CAN_INT 4

INA3221 ina3221;

MCP_CAN CAN(CAN_CS);

void setup() {

Serial.begin(115200);

Wire.begin();

SPI.begin();

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

ina3221.begin();

}

void loop() {

float voltage = ina3221.getBusVoltage_V(1);

float current = ina3221.getCurrent_mA(1) / 1000.0;

byte data[8];

memcpy(&data[0], &voltage, 4);

memcpy(&data[4], &current, 4);

CAN.sendMsgBuf(0x104, 0, 8, data);

Serial.printf("Sent Voltage: %.2f V, Current: %.2f A\n", voltage, current);

delay(1000);

}

Receiver:

#include <SPI.h>

#include <mcp_can.h>

#include <TFT_eSPI.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

TFT_eSPI tft = TFT_eSPI();

void setup() {

Serial.begin(115200);

SPI.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

if (CAN.checkReceive() == CAN_MSGAVAIL) {

unsigned long rxId;

byte len;

byte buf[8];

if (CAN.readMsgBuf(&rxId, &len, buf) == CAN_OK && rxId == 0x104) {

float voltage, current;

memcpy(&voltage, &buf[0], 4);

memcpy(&current, &buf[4], 4);

tft.fillScreen(TFT_BLACK);

tft.drawString("Voltage: " + String(voltage, 2) + " V", 10, 20);

tft.drawString("Current: " + String(current, 2) + " A", 10, 50);

Serial.printf("Received Voltage: %.2f V, Current: %.2f A\n", voltage, current);

}

}

}

Transmitter serial output:

Sent Voltage: 12.34 V, Current: 0.56 A

Sent Voltage: 12.30 V, Current: 0.57 A

...

Receiver serial output:

Received Voltage: 12.34 V, Current: 0.56 A

Received Voltage: 12.30 V, Current: 0.57 A

...

Receiver TFT display:

Voltage: 12.34 V

Current: 0.56 A

## CLASS 14 - NTC + TFT + CAN (N+T+C)

Transmitter:

#include <SPI.h>

#include <mcp_can.h>

#include <TFT_eSPI.h>

#define CAN_CS 5

#define CAN_INT 4

#define NTC_PIN A0

MCP_CAN CAN(CAN_CS);

float readNTCTemperature() {

int raw = analogRead(NTC_PIN);

float resistance = (1023.0 / raw - 1.0) * 10000;

float tempK = 1.0 / (0.001129148 + 0.000234125 * log(resistance) + 0.0000000876741 * pow(log(resistance), 3));

return tempK - 273.15;

}

void setup() {

Serial.begin(115200);

SPI.begin();

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

float temp = readNTCTemperature();

byte data[4];

memcpy(data, &temp, 4);

CAN.sendMsgBuf(0x105, 0, 4, data);

Serial.printf("Sent Temp: %.2f C\n", temp);

delay(1000);

}

Receiver:

#include <SPI.h>

#include <mcp_can.h>

#include <TFT_eSPI.h>

#define CAN_CS 5

#define CAN_INT 4

MCP_CAN CAN(CAN_CS);

TFT_eSPI tft = TFT_eSPI();

void setup() {

Serial.begin(115200);

SPI.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE);

tft.setTextSize(2);

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN Init Fail");

while(1);

}

CAN.setMode(MCP_NORMAL);

pinMode(CAN_INT, INPUT);

}

void loop() {

if (CAN.checkReceive() == CAN_MSGAVAIL) {

unsigned long rxId;

byte len;

byte buf[8];

if (CAN.readMsgBuf(&rxId, &len, buf) == CAN_OK && rxId == 0x105) {

float temp;

memcpy(&temp, &buf[0], 4);

tft.fillScreen(TFT_BLACK);

tft.drawString("Temp: " + String(temp, 2) + " C", 10, 20);

Serial.printf("Received Temp: %.2f C\n", temp);

}

}

}

Transmitter serial output:

Sent Temp: 25.62 C

Sent Temp: 25.60 C

...

Receiver serial output:

Received Temp: 25.62 C

Received Temp: 25.60 C

...

Receiver TFT display:

Temp: 25.62 C

## CLASS 15 -MAIN CODE(All Sensor Combination With Logic):

TRANSMITTER:

#include <Wire.h>

#include <INA3221.h>

#include <driver/adc.h>

#include <esp_adc_cal.h>

#include <math.h>

// === CAN Bus Libraries ===

#include <SPI.h>

#include <mcp2515.h>  // <-- Already included

// === WiFi Manager & OTA ===

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

// === System Configuration ===

#define SERIAL_SPEED 115200

#define SHUNT_RESISTOR_MOHM 100

#define RELAY_CHARGER_PIN 22

#define RELAY_MOTOR_PIN   21

#define VOLTAGE_THRESHOLD_HIGH 12.10

#define VOLTAGE_THRESHOLD_LOW  10.20

#define THERMISTOR_PIN 34

#define SERIES_RESISTOR 10000.0

#define NOMINAL_RESISTANCE 10000.0

#define NOMINAL_TEMPERATURE 25.0

#define B_COEFFICIENT 3950.0

#define VREF 1100.0

#define TEMPERATURE_LIMIT 50.0

#define I2C_SDA 16

#define I2C_SCL 17

// === INA3221 ===

TwoWire customWire = TwoWire(0);

INA3221 ina_0(INA3221_ADDR40_GND);

// === CAN Setup ===

MCP2515 mcp2515(5);  // CS pin for MCP2515 (adjust if needed)

struct can_frame canMsg;

bool relayChargingState = true;

bool motorRunning = true;

// === Thermistor Calibration ===

esp_adc_cal_characteristics_t adc_chars;

void current_measure_init() {

customWire.begin(I2C_SDA, I2C_SCL, 100000);

ina_0.begin(&customWire);

ina_0.reset();

ina_0.setShuntRes(SHUNT_RESISTOR_MOHM, SHUNT_RESISTOR_MOHM, SHUNT_RESISTOR_MOHM);

}

float readMotorTemperatureCelsius() {

uint32_t raw_adc = 0;

for (int i = 0; i < 64; i++) raw_adc += adc1_get_raw(ADC1_CHANNEL_6);

raw_adc /= 64;

uint32_t voltage_mv = esp_adc_cal_raw_to_voltage(raw_adc, &adc_chars);

float voltage = voltage_mv / 1000.0;

float resistance = SERIES_RESISTOR * voltage / (3.3 - voltage);

float steinhart = resistance / NOMINAL_RESISTANCE;

steinhart = log(steinhart);

steinhart /= B_COEFFICIENT;

steinhart += 1.0 / (NOMINAL_TEMPERATURE + 273.15);

steinhart = 1.0 / steinhart;

float tempC = steinhart - 273.15;

Serial.printf("Motor Temp: %.2f °C | Resistance: %.2f kΩ\n", tempC, resistance / 1000.0);

return tempC;

}

void setup() {

Serial.begin(SERIAL_SPEED);

delay(1000);

// === WiFi Manager ===

WiFi.mode(WIFI_STA);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("KIT4.2-Transmitter")) {

Serial.println("Failed to connect WiFi. Restarting...");

ESP.restart();

}

Serial.println("WiFi connected: " + WiFi.localIP().toString());

// === OTA Setup ===

ArduinoOTA.setHostname("EV-System_transmitter");

ArduinoOTA.begin();

Serial.println("OTA Ready");

pinMode(RELAY_CHARGER_PIN, OUTPUT);

pinMode(RELAY_MOTOR_PIN, OUTPUT);

digitalWrite(RELAY_CHARGER_PIN, LOW);

digitalWrite(RELAY_MOTOR_PIN, LOW);

relayChargingState = true;

motorRunning = true;

current_measure_init();

adc1_config_width(ADC_WIDTH_BIT_12);

adc1_config_channel_atten(ADC1_CHANNEL_6, ADC_ATTEN_DB_11);

esp_adc_cal_characterize(ADC_UNIT_1, ADC_ATTEN_DB_11, ADC_WIDTH_BIT_12, VREF, &adc_chars);

// === CAN Init ===

SPI.begin();

if (mcp2515.reset() != MCP2515::ERROR_OK) {

Serial.println("MCP2515 init failed");

while (1);

}

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

Serial.println("MCP2515 initialized");

Serial.println("System Initialized.");

}

void loop() {

ArduinoOTA.handle();  // === OTA Handler ===

float voltageA1 = ina_0.getVoltage(INA3221_CH1);

float currentA1 = ina_0.getCurrent(INA3221_CH1) * 1000;

float voltageA2 = ina_0.getVoltage(INA3221_CH2);

float currentA2 = ina_0.getCurrent(INA3221_CH2) * 1000;

float voltageB1 = ina_0.getVoltage(INA3221_CH3);

float currentB1 = ina_0.getCurrent(INA3221_CH3) * 1000;

Serial.printf("Charger (A1): %6.0f mA | %4.2f V\n", currentA1, voltageA1);

Serial.printf("Battery (A2): %6.0f mA | %4.2f V\n", currentA2, voltageA2);

Serial.printf("Motor   (B1): %6.0f mA | %4.2f V\n", currentB1, voltageB1);

if (relayChargingState && voltageA2 >= VOLTAGE_THRESHOLD_HIGH) {

relayChargingState = false;

digitalWrite(RELAY_CHARGER_PIN, HIGH);

Serial.println("🔋 Battery full — Charging OFF");

} else if (!relayChargingState && voltageA2 <= VOLTAGE_THRESHOLD_LOW) {

relayChargingState = true;

digitalWrite(RELAY_CHARGER_PIN, LOW);

Serial.println("🔋 Battery low — Charging ON");

}

float motorTemp = readMotorTemperatureCelsius();

if (motorTemp >= TEMPERATURE_LIMIT && motorRunning) {

digitalWrite(RELAY_MOTOR_PIN, HIGH);

motorRunning = false;

Serial.println("🔥 Motor temp too high — Motor OFF");

} else if (motorTemp < (TEMPERATURE_LIMIT - 5) && !motorRunning) {

digitalWrite(RELAY_MOTOR_PIN, LOW);

motorRunning = true;

Serial.println("✅ Motor cooled — Motor ON");

}

Serial.printf("Relay (Charger): %s | Relay (Motor): %s\n",

relayChargingState ? "ON (Charging)" : "OFF",

motorRunning ? "ON (Running)" : "OFF");

Serial.println("-----------------------------");

// === SEND SENSOR DATA OVER CAN ===

int temp_scaled     = (int)(motorTemp * 10);

int battV_scaled    = (int)(voltageA2 * 100);

int battC_scaled    = (int)currentA2;

int chargV_scaled   = (int)(voltageA1 * 100);

int chargC_scaled   = (int)currentA1;

int motorV_scaled   = (int)(voltageB1 * 100);

int motorC_scaled   = (int)currentB1;

uint8_t relays      = (relayChargingState ? 1 : 0) | (motorRunning ? 2 : 0);

// Frame 1: Temp, Batt V, Batt C

canMsg.can_id  = 0x101;

canMsg.can_dlc = 8;

canMsg.data[0] = temp_scaled >> 8;

canMsg.data[1] = temp_scaled & 0xFF;

canMsg.data[2] = battV_scaled >> 8;

canMsg.data[3] = battV_scaled & 0xFF;

canMsg.data[4] = battC_scaled >> 8;

canMsg.data[5] = battC_scaled & 0xFF;

canMsg.data[6] = 0;

canMsg.data[7] = 0;

mcp2515.sendMessage(&canMsg);

// Frame 2: Charger V/C + Motor V

canMsg.can_id  = 0x102;

canMsg.data[0] = chargV_scaled >> 8;

canMsg.data[1] = chargV_scaled & 0xFF;

canMsg.data[2] = chargC_scaled >> 8;

canMsg.data[3] = chargC_scaled & 0xFF;

canMsg.data[4] = motorV_scaled >> 8;

canMsg.data[5] = motorV_scaled & 0xFF;

canMsg.data[6] = 0;

canMsg.data[7] = 0;

mcp2515.sendMessage(&canMsg);

// Frame 3: Motor C + Relay States

canMsg.can_id  = 0x103;

canMsg.data[0] = motorC_scaled >> 8;

canMsg.data[1] = motorC_scaled & 0xFF;

canMsg.data[2] = relays;

canMsg.data[3] = 0;

canMsg.data[4] = 0;

canMsg.data[5] = 0;

canMsg.data[6] = 0;

canMsg.data[7] = 0;

mcp2515.sendMessage(&canMsg);

delay(2000);

}

RECEIVER:

#include <SPI.h>

#include <mcp2515.h>

#include "TFT_22_ILI9225.h"

#include <../fonts/FreeSans9pt7b.h>

#include <WiFi.h>

#include <HTTPClient.h>

#include <ArduinoJson.h>

#include <WiFiClientSecure.h>

#include <WiFiManager.h>        // WiFi Manager

#include <ArduinoOTA.h>         // OTA

// ===== TFT Pins and Config =====

#define TFT_RST 26

#define TFT_RS  25

#define TFT_CS  15

#define TFT_SDI 13

#define TFT_CLK 14

#define TFT_LED 0

#define TFT_BRIGHTNESS 200

// ===== Network & Server Config =====

const char* kitID = "kit7";

const char* flaskAPI = "http://192.168.1.130:5010/api/v1/data";

const char* thingsboardURL = "https://192.168.1.238:8080/api/v1/esp4.2/telemetry";

// ===== TFT SPI and Object =====

SPIClass hspi(HSPI);

TFT_22_ILI9225 tft = TFT_22_ILI9225(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS);

// ===== CAN MCP2515 Config =====

MCP2515 mcp2515(5);  // CS pin

struct can_frame rxMsg;

// ===== Sensor Data Variables =====

float motorTempC = NAN;

float batteryVolt = NAN;

int16_t batteryCurrent = 0;

float chargerVolt = NAN;

int16_t chargerCurrent = 0;

float motorVolt = NAN;

int16_t motorCurrent = 0;

bool relayCharging = false;

bool relayMotor = false;

bool has101 = false, has102 = false, has103 = false;

void setup() {

Serial.begin(115200);

delay(1000);

// ===== WiFiManager Setup =====

WiFiManager wm;

bool res = wm.autoConnect("KIT4.2-Receiver");

if (!res) {

Serial.println("Failed to connect. Restarting...");

delay(3000);

ESP.restart();

}

Serial.println("WiFi connected: " + WiFi.localIP().toString());

// ===== OTA Setup =====

ArduinoOTA.setHostname("EV-System_Receiver");

ArduinoOTA.onStart([]() {

Serial.println("Start updating...");

});

ArduinoOTA.onEnd([]() {

Serial.println("\nUpdate Finished.");

});

ArduinoOTA.onProgress([](unsigned int progress, unsigned int total) {

Serial.printf("Progress: %u%%\r", (progress / (total / 100)));

});

ArduinoOTA.onError([](ota_error_t error) {

Serial.printf("Error[%u]: ", error);

if (error == OTA_AUTH_ERROR) Serial.println("Auth Failed");

else if (error == OTA_BEGIN_ERROR) Serial.println("Begin Failed");

else if (error == OTA_CONNECT_ERROR) Serial.println("Connect Failed");

else if (error == OTA_RECEIVE_ERROR) Serial.println("Receive Failed");

else if (error == OTA_END_ERROR) Serial.println("End Failed");

});

ArduinoOTA.begin();

// ===== Init CAN MCP2515 =====

SPI.begin();

if (mcp2515.reset() != MCP2515::ERROR_OK) {

Serial.println("MCP2515 Reset failed!");

while (1);

}

mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

// ===== Init TFT =====

hspi.begin(TFT_CLK, -1, TFT_SDI, -1);

tft.begin(hspi);

tft.setOrientation(0);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 20, "EV-System", COLOR_YELLOW);

}

void displayData() {

tft.fillRectangle(0, 28, 176, 220, COLOR_BLACK);

int16_t labelX = 5;

int16_t valueX = 100;

int16_t y = 40;

int16_t lineSpacing = 22;

tft.drawGFXText(labelX, y, "Motor T:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(motorTempC) ? "---" : String(motorTempC, 1) + " C", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Batt V:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(batteryVolt) ? "---" : String(batteryVolt, 2) + " V", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Batt I:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(batteryCurrent) + " mA", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Chg Volt:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(chargerVolt) ? "---" : String(chargerVolt, 2) + " V", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Chg Curr:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(chargerCurrent) + " mA", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Motor V:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(motorVolt) ? "---" : String(motorVolt, 2) + " V", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Motor I:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(motorCurrent) + " mA", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Relay Chg:", COLOR_CYAN);

tft.drawGFXText(valueX, y, relayCharging ? "ON" : "OFF", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Relay Mtr:", COLOR_CYAN);

tft.drawGFXText(valueX, y, relayMotor ? "ON" : "OFF", COLOR_WHITE);

}

void sendToFlask() {

WiFiClient client;

HTTPClient http;

if (http.begin(client, flaskAPI)) {

http.addHeader("Content-Type", "application/json");

StaticJsonDocument<512> doc;

doc["kit_id"] = kitID;

doc["motor_temp"] = motorTempC;

doc["battery_voltage"] = batteryVolt;

doc["battery_current"] = batteryCurrent;

doc["charger_voltage"] = chargerVolt;

doc["charger_current"] = chargerCurrent;

doc["motor_voltage"] = motorVolt;

doc["motor_current"] = motorCurrent;

doc["relay_charging"] = relayCharging;

doc["relay_motor"] = relayMotor;

String payload;

serializeJson(doc, payload);

int code = http.POST(payload);

Serial.printf("Flask POST Code: %d\n", code);

http.end();

} else {

Serial.println("Flask API connection failed.");

}

}

void sendToThingsBoard() {

WiFiClientSecure client;

client.setInsecure();  // Insecure mode for test; use proper certificate in production

HTTPClient https;

if (https.begin(client, thingsboardURL)) {

https.addHeader("Content-Type", "application/json");

StaticJsonDocument<512> doc;

doc["motor_temp"] = motorTempC;

doc["battery_voltage"] = batteryVolt;

doc["battery_current"] = batteryCurrent;

doc["charger_voltage"] = chargerVolt;

doc["charger_current"] = chargerCurrent;

doc["motor_voltage"] = motorVolt;

doc["motor_current"] = motorCurrent;

doc["relay_charging"] = relayCharging;

doc["relay_motor"] = relayMotor;

String payload;

serializeJson(doc, payload);

int code = https.POST(payload);

Serial.printf("ThingsBoard POST Code: %d\n", code);

https.end();

} else {

Serial.println("ThingsBoard connection failed.");

}

}

void loop() {

ArduinoOTA.handle();  // OTA handler

if (mcp2515.readMessage(&rxMsg) == MCP2515::ERROR_OK) {

switch (rxMsg.can_id) {

case 0x101:

motorTempC     = ((int16_t)(rxMsg.data[0] << 8 | rxMsg.data[1])) / 10.0;

batteryVolt    = ((int16_t)(rxMsg.data[2] << 8 | rxMsg.data[3])) / 100.0;

batteryCurrent = (int16_t)(rxMsg.data[4] << 8 | rxMsg.data[5]);

has101 = true;

break;

case 0x102:

chargerVolt    = ((int16_t)(rxMsg.data[0] << 8 | rxMsg.data[1])) / 100.0;

chargerCurrent = (int16_t)(rxMsg.data[2] << 8 | rxMsg.data[3]);

motorVolt      = ((int16_t)(rxMsg.data[4] << 8 | rxMsg.data[5])) / 100.0;

has102 = true;

break;

case 0x103:

motorCurrent   = (int16_t)(rxMsg.data[0] << 8 | rxMsg.data[1]);

uint8_t relayBits = rxMsg.data[2];

relayCharging = relayBits & 0x01;

relayMotor    = relayBits & 0x02;

has103 = true;

break;

}

if (has101 && has102 && has103) {

displayData();

sendToFlask();

sendToThingsBoard();

has101 = has102 = has103 = false;

}

}

delay(100);  // Small delay to avoid busy loop

}

Transmitter Serial Output (Sending data over CAN):

WiFi connected: 192.168.1.45

OTA Ready

MCP2515 initialized

System Initialized.

Charger (A1):   1500 mA | 12.35 V

Battery (A2):   1800 mA | 12.05 V

Motor   (B1):    700 mA | 11.90 V

Motor Temp: 37.25 °C | Resistance: 12.35 kΩ

Relay (Charger): ON (Charging) | Relay (Motor): ON (Running)

-----------------------------

Charger (A1):   1400 mA | 12.40 V

Battery (A2):   1750 mA | 12.15 V

Motor   (B1):    680 mA | 11.85 V

Motor Temp: 37.10 °C | Resistance: 12.32 kΩ

Relay (Charger): ON (Charging) | Relay (Motor): ON (Running)

-----------------------------

Battery full — Charging OFF

Charger (A1):   1300 mA | 12.45 V

Battery (A2):   2100 mA | 12.15 V

Motor   (B1):    650 mA | 11.80 V

Motor Temp: 37.00 °C | Resistance: 12.30 kΩ

Relay (Charger): OFF | Relay (Motor): ON (Running)

-----------------------------

Motor temp too high — Motor OFF

Charger (A1):   1200 mA | 12.50 V

Battery (A2):   2200 mA | 12.10 V

Motor   (B1):    950 mA | 60.00 V

Motor Temp: 51.25 °C | Resistance: 11.50 kΩ

Relay (Charger): OFF | Relay (Motor): OFF

-----------------------------

Motor cooled — Motor ON

Charger (A1):   1100 mA | 12.48 V

Battery (A2):   2000 mA | 12.08 V

Motor   (B1):    700 mA | 11.90 V

Motor Temp: 45.00 °C | Resistance: 12.00 kΩ

Relay (Charger): OFF | Relay (Motor): ON (Running)

-----------------------------

Receiver Serial Output (Receiving CAN data, displaying & sending to Flask & ThingsBoard):

WiFi connected: 192.168.1.55

Start updating...

Update Finished.

MCP2515 Reset failed! (if reset failed, else nothing)

[Receiving CAN ID: 0x101] Motor Temp: 37.3 C | Batt V: 12.05 V | Batt I: 1800 mA

[Receiving CAN ID: 0x102] Charger V: 12.35 V | Charger I: 1500 mA | Motor V: 11.90 V

[Receiving CAN ID: 0x103] Motor I: 700 mA | Relay Chg: ON | Relay Mtr: ON

Flask POST Code: 200

ThingsBoard POST Code: 200

[Receiving CAN ID: 0x101] Motor Temp: 51.2 C | Batt V: 12.10 V | Batt I: 2200 mA

[Receiving CAN ID: 0x102] Charger V: 12.50 V | Charger I: 1200 mA | Motor V: 11.85 V

[Receiving CAN ID: 0x103] Motor I: 950 mA | Relay Chg: OFF | Relay Mtr: OFF

Flask POST Code: 200

ThingsBoard POST Code: 200

[Receiving CAN ID: 0x101] Motor Temp: 45.0 C | Batt V: 12.08 V | Batt I: 2000 mA

[Receiving CAN ID: 0x102] Charger V: 12.48 V | Charger I: 1100 mA | Motor V: 11.90 V

[Receiving CAN ID: 0x103] Motor I: 700 mA | Relay Chg: OFF | Relay Mtr: ON

Flask POST Code: 200

ThingsBoard POST Code: 200

TFT Display would show (updated every received message):

Motor T: 37.3 C

Batt V: 12.05 V

Batt I: 1800 mA

Chg Volt: 12.35 V

Chg Curr: 1500 mA

Motor V: 11.90 V

Motor I: 700 mA

Relay Chg: ON

Relay Mtr: ON