Energy Monitoring System

(Using ESP32 & PZEM004T with TFT Display)

User Manual

| Filling Reference : | Integra/AMU/KIT1/User Manual |  |
| --- | --- | --- |
| Document Title : | User Manual |  |
| Version : | Draft |  |
| Document Author: | Mr. Sabarish |  |
| Date Created : | 01/06/2025 |  |
| Date Modified : | 01/06/2025 |  |

|  | Reviewed By | Accepted By |
| --- | --- | --- |
| Signature & Date |  |  |
| Name | Mr. Bala |  |
| Designation | Technical Consultant |  |

Change History:

| Version | Date | Author | Change History |
| --- | --- | --- | --- |
| Draft | 01/06/2025 | Mr. Sabarish | Initial Draft Created |

Distribution List:

| Name | Email Address |
| --- | --- |
| Mr. Bala | bala@integratech.com.sg |
| Mr. Hari | hari@integratech.com.sg |
| Mr. Selva Pandian | selvapandian@integratech.com.sg |
| Mr. Sabarish | sabarish@integratech.com.sg |
| Mr. Sivaguru | sivaguru@integratech.com.sg |

Table of Contents

| Contents 1. Project Overview 4 2. Hardware Component 4 3. Working Principle 5 4. Pin Connections 6 5. Software Components 8 6. Data Flow 8 7. API Endpoints 9 8. Schematic Diagram 9 9. Conclusion 10 10. Code Examples 10 Class 1 – TFT "Welcome to IoT Lab" Message 10 Class 2 – Display Voltage Only 11 Class 3 – Full PZEM Data on TFT Display 12 Class 4 – OTA Setup with Display 14 Class 5 – Send Data to ThingsBoard 16 Class 6 – Send Data to Flask API (MongoDB) 17 Class 7 – Final Code – Full Integration. 19 |  |
| --- | --- |

- Project Overview

This project monitors real-time electrical parameters using the PZEM004T V3.0 sensor module and displays the data on a 2.2" ILI9225 TFT screen. The ESP32 microcontroller reads the sensor data, displays it, and transmits it to both a Flask API (with a MongoDB backend) and the ThingsBoard IoT platform.

- Hardware Component

| Component | Description |
| --- | --- |
| ESP32 Dev Board | Main microcontroller with Wi-Fi capability |
| PZEM004T V3.0 | Energy meter sensor for voltage, current, power |
| ILI9225 TFT 2.2" | SPI-based color display |
| Jumper Wires | For electrical connections |
| Power Supply | 5V regulated power for ESP32 and peripherals |

- Working Principle

- The PZEM004T V3.0 sensor measures AC voltage, current, power, frequency, and power factor.

- It communicates with the ESP32 via UART (Serial2) using the Modbus RTU protocol.

- The ESP32:

  - Collects data every 500 ms.

  - Displays it on the TFT screen.

  - Sends it to:

    - ThingsBoard IoT Cloud via HTTPS.

    - Flask API for local MongoDB storage.

- Wi-Fi Manager simplifies initial Wi-Fi configuration via a captive portal.

- Arduino OTA enables over-the-air firmware updates.

- Pin Connections

TFT Display (ILI9225) ↔ ESP32

| TFT Pin | ESP32 Pin | Description |
| --- | --- | --- |
| VCC | 3.3V | Power |
| GND | GND | Ground |
| CLK | GPIO14 | SPI Clock |
| SDA | GPIO13 | SPI MOSI (Data) |
| RS | GPIO25 | Register Select |
| RST | GPIO26 | Reset |
| CS | GPIO15 | Chip Select |
| LED | GPIO0 | Backlight Control |

PZEM004T V3.0 ↔ ESP32

| PZEM Pin | ESP32 Pin | Description |
| --- | --- | --- |
| TX | GPIO16 | Transmit to ESP32 |
| RX | GPIO17 | Receive from ESP32 |
| GND | GND | Common Ground |
| VCC | 5V | Sensor Power |

- Software Components

Developed using the Arduino IDE with the following libraries:

- TFT_22_ILI9225 – for TFT display control

- Wi-Fi Manager – for Wi-Fi configuration

- PZEM004T v30 – for interfacing with the PZEM sensor

- Arduino Json – for formatting data for HTTP POST

- Arduino OTA – for OTA firmware updates

- HTTP Client & Wi-Fi-Client Secure – for HTTP/HTTPS communication

- Data Flow

- PZEM004T sends electrical parameters via UART.

- ESP32 parses and displays the data.

- Data is transmitted to:

  - ThingsBoard via HTTPS.

  - Flask API via HTTP.

- Flask API stores the data in MongoDB.

- API Endpoints

- ThingsBoard:

https://192.168.1.238:8080/api/v1/esp1/telemetry

(Token-based endpoint for telemetry upload)

- Flask API:

http://192.168.1.130:5010/api/v1/data

(Accepts JSON payload and stores it in MongoDB)

- Schematic Diagram

[PZEM004T] [ESP32] [TFT Display] TX ------> RX2 (GPIO16) RX <------ TX2 (GPIO17) GND ------> GND GND ------> GND VCC ------> 5V VCC ------> 3.3V CLK ------> GPIO14 SDA ------> GPIO13 RS ------> GPIO25 RST ------> GPIO26 CS ------> GPIO15

- Conclusion

This project offers a cost-effective and scalable solution for smart energy monitoring. With real-time display, cloud integration, and OTA capabilities, it is well-suited for both residential and industrial energy management applications.

- Code Examples

## Class 1 – TFT "Welcome to IoT Lab" Message

#include "SPI.h"

#include "TFT_22_ILI9225.h"

#include <WiFiManager.h>

#include <../fonts/FreeSans9pt7b.h>

#define TFT_RST 26

#define TFT_RS  25

#define TFT_CS  15

#define TFT_SDI 13

#define TFT_CLK 14

#define TFT_LED 0

#define TFT_BRIGHTNESS 200

SPIClass hspi(HSPI);

TFT_22_ILI9225 tft = TFT_22_ILI9225(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS);

void setup() {

Serial.begin(115200);

WiFiManager wm;

if (!wm.autoConnect("TFT-Welcome")) {

Serial.println("WiFi failed");

ESP.restart();

}

hspi.begin();

tft.begin(hspi);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 60, "Welcome to", COLOR_CYAN);

tft.drawGFXText(10, 90, "IoT Lab!", COLOR_YELLOW);

}

void loop() {}

TFT Display Output:

+----------------------------+

|                               	|

|   Welcome to 		|

|   IoT Lab!              	|

|                         		|

+----------------------------+

Serial Monitor:

*If WiFi connects successfully, you’ll see:*

[WiFiManager] AutoConnect: TFT-Welcome

Connected to WiFi: 192.168.1.xxx

## Class 2 – Display Voltage Only

#include "SPI.h"

#include "TFT_22_ILI9225.h"

#include <PZEM004Tv30.h>

#include <WiFiManager.h>

#include <../fonts/FreeSans9pt7b.h>

#define TFT_RST 26

#define TFT_RS  25

#define TFT_CS  15

#define TFT_SDI 13

#define TFT_CLK 14

#define TFT_LED 0

#define TFT_BRIGHTNESS 200

SPIClass hspi(HSPI);

TFT_22_ILI9225 tft = TFT_22_ILI9225(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS);

PZEM004Tv30 pzem(&Serial2, 16, 17);

void setup() {

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, 16, 17);

WiFiManager wm;

if (!wm.autoConnect("Voltage-Only")) {

ESP.restart();

}

hspi.begin();

tft.begin(hspi);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

}

void loop() {

float voltage = pzem.voltage();

tft.fillRectangle(0, 28, 176, 132, COLOR_BLACK);

tft.drawGFXText(10, 60, "Voltage:", COLOR_CYAN);

tft.drawGFXText(100, 60, isnan(voltage) ? "---" : String(voltage, 1) + " V", COLOR_WHITE);

delay(1000);

}

TFT Display Output (example with voltage 231.4V):

+-------------------------+

|                         		|

|   Voltage:  231.4 V     	|

|                        		|

+-------------------------+

Serial Monitor:

*WiFiManager Portal Output*

Voltage: 231.4

## Class 3 – Full PZEM Data on TFT Display

#include "SPI.h"

#include "TFT_22_ILI9225.h"

#include <PZEM004Tv30.h>

#include <WiFiManager.h>

#include <../fonts/FreeSans9pt7b.h>

#define TFT_RST 26

#define TFT_RS  25

#define TFT_CS  15

#define TFT_SDI 13

#define TFT_CLK 14

#define TFT_LED 0

#define TFT_BRIGHTNESS 200

SPIClass hspi(HSPI);

TFT_22_ILI9225 tft = TFT_22_ILI9225(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS);

PZEM004Tv30 pzem(&Serial2, 16, 17);

void setup() {

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, 16, 17);

WiFiManager wm;

if (!wm.autoConnect("Full-PZEM-Display")) {

Serial.println("WiFi Failed!");

ESP.restart();

}

hspi.begin();

tft.begin(hspi);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 20, "Energy Monitor", COLOR_YELLOW);

}

void loop() {

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float frequency = pzem.frequency();

float pf = pzem.pf();

tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);

int16_t labelX = 5;

int16_t valueX = 100;

int16_t y = 45;

int16_t spacing = 24;

tft.drawGFXText(labelX, y, "Voltage:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(voltage) ? "---" : String(voltage, 1) + " V", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "Current:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(current) ? "---" : String(current, 2) + " A", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "Power:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(power) ? "---" : String(power, 1) + " W", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "Freq:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(frequency) ? "---" : String(frequency, 1) + " Hz", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "P.F:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(pf) ? "---" : String(pf, 2), COLOR_WHITE);

delay(1000);

}

TFT Display Output:

+------------------------------+

| Energy Monitor                |

|                             	    |

| Voltage:   231.4 V            |

| Current:     1.25 A            |

| Power:     285.0 W           |

| Freq:       50.0 Hz              |

| P.F:         0.96                     |

+------------------------------+

Serial Monitor:

WiFi Connected

Voltage: 231.4 V | Current: 1.25 A | Power: 285.0 W | Freq: 50.0 Hz | PF: 0.96

## Class 4 – OTA Setup with Display

#include "SPI.h"

#include "TFT_22_ILI9225.h"

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <../fonts/FreeSans9pt7b.h>

#define TFT_RST 26

#define TFT_RS  25

#define TFT_CS  15

#define TFT_SDI 13

#define TFT_CLK 14

#define TFT_LED 0

#define TFT_BRIGHTNESS 200

SPIClass hspi(HSPI);

TFT_22_ILI9225 tft = TFT_22_ILI9225(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS);

void setup() {

Serial.begin(115200);

WiFiManager wm;

if (!wm.autoConnect("OTA-Display")) {

Serial.println("WiFi Failed");

ESP.restart();

}

Serial.println("Connected to WiFi: " + WiFi.localIP().toString());

hspi.begin();

tft.begin(hspi);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 50, "OTA Ready", COLOR_YELLOW);

ArduinoOTA.setHostname("ESP32_Energy");

ArduinoOTA.onStart([]() {

Serial.println("OTA Start");

});

ArduinoOTA.onEnd([]() {

Serial.println("\nOTA End");

});

ArduinoOTA.onProgress([](unsigned int progress, unsigned int total) {

Serial.printf("OTA Progress: %u%%\r", (progress * 100) / total);

});

ArduinoOTA.onError([](ota_error_t error) {

Serial.printf("OTA Error[%u]: ", error);

if (error == OTA_AUTH_ERROR) Serial.println("Auth Failed");

else if (error == OTA_BEGIN_ERROR) Serial.println("Begin Failed");

else if (error == OTA_CONNECT_ERROR) Serial.println("Connect Failed");

else if (error == OTA_RECEIVE_ERROR) Serial.println("Receive Failed");

else if (error == OTA_END_ERROR) Serial.println("End Failed");

});

ArduinoOTA.begin();

}

void loop() {

ArduinoOTA.handle();

}

📟 TFT Display Output:

+-------------------------+

|                         |

|       OTA Ready         |

|                         |

+-------------------------+

Serial Monitor Output:

WiFi connected: 192.168.1.xxx

OTA Ready

During OTA Upload:

OTA Start

OTA Progress: 10%

...

OTA Progress: 100%

OTA End

## Class 5 – Send Data to ThingsBoard

#include "PZEM004Tv30.h"

#include <WiFiManager.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <ArduinoJson.h>

const char* thingsboardURL = "https://192.168.1.238:8080/api/v1/esp1/telemetry";

PZEM004Tv30 pzem(&Serial2, 16, 17);

void setup() {

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, 16, 17);

WiFiManager wm;

if (!wm.autoConnect("ThingsBoard-Post")) {

ESP.restart();

}

Serial.println("WiFi Connected");

}

void loop() {

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float frequency = pzem.frequency();

float pf = pzem.pf();

WiFiClientSecure client;

client.setInsecure();

HTTPClient https;

if (https.begin(client, thingsboardURL)) {

https.addHeader("Content-Type", "application/json");

StaticJsonDocument<256> doc;

doc["voltage"] = voltage;

doc["current"] = current;

doc["power"] = power;

doc["frequency"] = frequency;

doc["pf"] = pf;

String json;

serializeJson(doc, json);

int code = https.POST(json);

Serial.printf("ThingsBoard POST Code: %d\n", code);

https.end();

} else {

Serial.println("ThingsBoard connection failed.");

}

delay(5000);

}

Serial Monitor Output:

WiFi Connected

Voltage: 230.5 V | Current: 1.10 A | Power: 253.6 W | Freq: 50.1 Hz | PF: 0.95

ThingsBoard POST Code: 200

ThingsBoard Dashboard: (You’ll see telemetry updates)

Voltage	Current	Power	Frequency	PF

230.5	1.10	253.6	50.1	0.95

## Class 6 – Send Data to Flask API (MongoDB)

#include "PZEM004Tv30.h"

#include <WiFiManager.h>

#include <HTTPClient.h>

#include <WiFiClient.h>

#include <ArduinoJson.h>

const char* flaskAPI = "http://192.168.1.130:5010/api/v1/data";

const char* kitID = "kit1";

PZEM004Tv30 pzem(&Serial2, 16, 17);

void setup() {

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, 16, 17);

WiFiManager wm;

if (!wm.autoConnect("Flask-API-Post")) {

ESP.restart();

}

Serial.println("WiFi Connected");

}

void loop() {

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float frequency = pzem.frequency();

float pf = pzem.pf();

WiFiClient httpClient;

HTTPClient http;

if (http.begin(httpClient, flaskAPI)) {

http.addHeader("Content-Type", "application/json");

StaticJsonDocument<300> doc;

doc["kit_id"] = kitID;

doc["voltage"] = voltage;

doc["current"] = current;

doc["power"] = power;

doc["frequency"] = frequency;

doc["pf"] = pf;

String json;

serializeJson(doc, json);

int code = http.POST(json);

Serial.printf("Flask POST Code: %d\n", code);

http.end();

} else {

Serial.println("Flask API connection failed.");

}

delay(5000);

}

Serial Monitor Output:

WiFi Connected

Flask POST Code: 200

📨 Flask Server Receives JSON Payload:

json

{

"kit_id": "kit1",

"voltage": 230.5,

"current": 1.10,

"power": 253.6,

"frequency": 50.1,

"pf": 0.95

}

📦 MongoDB Entry Example:

json

{

"_id": ObjectId("..."),

"kit_id": "kit1",

"voltage": 230.5,

"current": 1.1,

"power": 253.6,

"frequency": 50.1,

"pf": 0.95,

"timestamp": "2025-06-05T10:45:00Z"

}

## Class 7 – Final Code – Full Integration.

#include "SPI.h"

#include "TFT_22_ILI9225.h"

#include <../fonts/FreeSans9pt7b.h>

#include <PZEM004Tv30.h>

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <ArduinoJson.h>

// TFT Pins

#define TFT_RST 26

#define TFT_RS  25

#define TFT_CS  15

#define TFT_SDI 13

#define TFT_CLK 14

#define TFT_LED 0

#define TFT_BRIGHTNESS 200

// Kit ID & URLs

const char* kitID = "kit1";

const char* thingsboardURL = "https://192.168.1.238:8080/api/v1/esp1/telemetry"; // Replace <YOUR_TOKEN>

const char* flaskAPI = "http://192.168.1.130:5010/api/v1/data";

// SPI & Display

SPIClass hspi(HSPI);

TFT_22_ILI9225 tft = TFT_22_ILI9225(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS);

// PZEM (Serial2: RX=16, TX=17)

PZEM004Tv30 pzem(&Serial2, 16, 17);

void setup() {

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, 16, 17);

// Display Setup

hspi.begin();

tft.begin(hspi);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 20, "Energy Monitor", COLOR_YELLOW);

// WiFiManager Portal

WiFiManager wm;

wm.setTimeout(180);  // 3 mins

if (!wm.autoConnect("Energy-Monitor")) {

Serial.println("Failed to connect. Restarting...");

ESP.restart();

}

Serial.println("WiFi connected: " + WiFi.localIP().toString());

// OTA Setup

ArduinoOTA.setHostname("EnergyMonitor");

ArduinoOTA.begin();

}

void loop() {

ArduinoOTA.handle();

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float frequency = pzem.frequency();

float pf = pzem.pf();

// Serial Debug

Serial.printf("Voltage: %.2f V | Current: %.2f A | Power: %.2f W | Freq: %.2f Hz | PF: %.2f\n",

voltage, current, power, frequency, pf);

// TFT Display Update

tft.fillRectangle(0, 28, 176, 192, COLOR_BLACK);

tft.setGFXFont(&FreeSans9pt7b);

int16_t labelX = 5;

int16_t valueX = 100;

int16_t y = 45;

int16_t spacing = 24;

tft.drawGFXText(labelX, y, "Voltage:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(voltage) ? "---" : String(voltage, 1) + " V", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "Current:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(current) ? "---" : String(current, 2) + " A", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "Power:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(power) ? "---" : String(power, 1) + " W", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "Freq:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(frequency) ? "---" : String(frequency, 1) + " Hz", COLOR_WHITE); y += spacing;

tft.drawGFXText(labelX, y, "P.F:", COLOR_CYAN);

tft.drawGFXText(valueX, y, isnan(pf) ? "---" : String(pf, 2), COLOR_WHITE);

// Send to ThingsBoard

WiFiClientSecure client;

client.setInsecure();

HTTPClient https;

if (https.begin(client, thingsboardURL)) {

https.addHeader("Content-Type", "application/json");

StaticJsonDocument<256> doc;

doc["voltage"] = voltage;

doc["current"] = current;

doc["power"] = power;

doc["frequency"] = frequency;

doc["pf"] = pf;

String json;

serializeJson(doc, json);

int code = https.POST(json);

Serial.printf("ThingsBoard POST Code: %d\n", code);

https.end();

} else {

Serial.println("ThingsBoard connection failed.");

}

// Send to Flask (MongoDB)

WiFiClient httpClient;

HTTPClient http;

if (http.begin(httpClient, flaskAPI)) {

http.addHeader("Content-Type", "application/json");

StaticJsonDocument<300> doc;

doc["kit_id"] = kitID;

doc["voltage"] = voltage;

doc["current"] = current;

doc["power"] = power;

doc["frequency"] = frequency;

doc["pf"] = pf;

String json;

serializeJson(doc, json);

int code = http.POST(json);

Serial.printf("Flask POST Code: %d\n", code);

http.end();

} else {

Serial.println("Flask API connection failed.");

}

delay(500);

}

TFT Display Output:

+------------------------------+

| Energy Monitor               |

|                             	   |

| Voltage:   230.5 V            |

| Current:     1.10 A            |

| Power:     253.6 W           |

| Freq:       50.1 Hz         	    |

| P.F:         0.95                     |

+------------------------------+

Serial Monitor:

WiFi connected: 192.168.1.xxx

Voltage: 230.50 V | Current: 1.10 A | Power: 253.60 W | Freq: 50.10 Hz | PF: 0.95

ThingsBoard POST Code: 200

Flask POST Code: 200