Environmental Monitoring System

(Using ESP32 & PZEM004T with TFT Display)

User Manual

| Filling Reference : | Integra/AMU/KIT3/User Manual |  |
| --- | --- | --- |
| Document Title : | User Manual |  |
| Version : | Draft |  |
| Document Author: | Mr. Sabarish |  |
| Date Created : | 01/06/2025 |  |
| Date Modified : | 01/06/2025 |  |

|  | Reviewed By | Accepted By |
| --- | --- | --- |
| Signature & Date |  |  |
| Name | Mr. Bala |  |
| Designation | Technical Consultant |  |

Change History:

| Version | Date | Author | Change History |
| --- | --- | --- | --- |
| Draft | 01/06/2025 | Mr. Sabarish | Initial Draft Created |

Distribution List:

| Name | Email Address |
| --- | --- |
| Mr. Bala | bala@integratech.com.sg |
| Mr. Hari | hari@integratech.com.sg |
| Mr. Selva Pandian | selvapandian@integratech.com.sg |
| Mr. Sabarish | sabarish@integratech.com.sg |
| Mr. Sivaguru | sivaguru@integratech.com.sg |

Table of Contents

| Contents 1. Project Overview 4 2. Objectives 4 3. Hardware Component 5 4. Software Components 6 5. Circuit Diagram and Wiring 7 6. Code Explanation 11 7. Setup and Usage Instructions 12 8. Data Flow and Communication 13 9. Trouble shooting and FAQs 15 10. Code Examples 16 CLASS 1 - ESP1: SHT20 , ESP2: PZEM004T 16 CLASS 2 - ESP1: SHT20, ESP2: VL53L0X 19 CLASS 3 - ESP1: SHT20, ESP2: PZEM004T + VL53L0X 22 CLASS 4 - ESP1: ADP810, ESP2: PZEM004T 25 CLASS #5 - ESP1: ADP810, ESP2: VL53L0X 28 CLASS 6 - ESP1: TFT, ESP2: PZEM004T 31 CLASS 7 - ESP1: TFT, ESP2: VL53L0X 34 CLASS 8 - ESP1: ADP810 + TFT, ESP2: PZEM004T 36 CLASS 9 - ESP1: ADP810 + TFT, ESP2: VL53L0X 40 CLASS 10 – Main Code 43 |  |
| --- | --- |

- Project Overview

This project implements an IoT-based environmental monitoring system designed to measure a range of environmental and electrical parameters, including air quality, temperature, humidity, atmospheric pressure, distance, and power metrics. The system architecture is based on two ESP32 microcontrollers operating in tandem:

- ESP32 #1 (Receiver): Collects data from various sensors, displays real-time information on a TFT screen, and transmits the data to cloud platforms for remote access and analysis.

- ESP32 #2 (Transmitter): Captures distance and electrical measurements, then transmits this data wirelessly to ESP32 #1.

This modular design enables scalable, flexible, and remote monitoring, making it suitable for diverse environmental and energy management applications.

- Objectives

- Design and develop a reliable multi-sensor platform for comprehensive environmental and electrical monitoring.

- Integrate a color TFT display to provide real-time, on-site visualization of sensor data.

- Enable seamless wireless data transmission and cloud-based storage for remote access and analysis.

- Implement an intuitive Wi-Fi configuration process using a captive portal for user-friendly setup.

- Support over-the-air (OTA) firmware updates to facilitate remote maintenance and upgrades.

- Ensure modularity and scalability by allowing straightforward integration of additional sensors.

- Hardware Component

| Component | Description | Interface | Notes |
| --- | --- | --- | --- |
| ESP32 Dev Board (x2) | Core microcontrollers | WiFi, UART, I2C, SPI | Primary processing units |
| SDS011 | Air quality sensor (PM2.5 and PM10) | UART | Serial data at 9600 baud |
| SHT20 (via Modbus RTU) | Temperature & Humidity sensor | UART | Modbus communication via RS-485 |
| ADP810 | Pressure sensor | I2C | Calibrated pressure readings |
| VL53L0X | Time-of-Flight distance sensor | I2C | Measures distance to object in cm |
| PZEM004T v3.0 | Electrical parameter sensor (Voltage, Current, Power, Energy) | UART | Reads AC parameters |
| TFT 2.2” ILI9225 | Color graphical display | SPI | Displays sensor data in real time |
| Power Supply | 5V/3.3V regulated power source | Power | Ensures stable voltage for sensors |
| Connecting wires | Jumper wires and connectors | N/A | For sensor and peripheral connections |

- Software Components

| Library/Framework | Purpose |
| --- | --- |
| WiFiManager | WiFi connection handling and captive portal setup |
| ArduinoOTA | OTA firmware update management |
| HTTPClient | HTTP client for POST requests |
| ArduinoJson | JSON serialization/deserialization |
| ModbusMaster | Modbus RTU communication with SHT20 |
| Adafruit_VL53L0X | VL53L0X sensor driver |
| PZEM004Tv30 | PZEM004T electrical sensor driver |
| TFT_22_ILI9225 | SPI TFT display driver |

- Circuit Diagram and Wiring

Textual Wiring Overview

ESP32 #1 Receiver:

| Sensor/Peripheral | ESP32 Pin(s) | Interface | Details |
| --- | --- | --- | --- |
| SDS011 (PM Sensor) | RX=16, TX=17 | UART1 | 9600 baud |
| SHT20 (Modbus RTU) | RX=18, TX=19 | UART2 | 9600 baud |
| ADP810 (Pressure) | SDA=33, SCL=32 | I2C (Wire1) | Calibrated sensor |
| TFT Display | RST=26, RS=25, CS=15, LED=0, CLK=14, MOSI=13 | SPI (HSPI) | SPI interface |
| Power Supply | 3.3V / GND | Power | Stable 3.3V |

ESP32 #2 Transmitter:

| Sensor/Peripheral | ESP32 Pin(s) | Interface | Details |
| --- | --- | --- | --- |
| VL53L0X Sensor | SDA=21, SCL=22 | I2C (Wire0) | Distance sensor |
| PZEM004T Electrical | RX=16, TX=17 | UART2 | 9600 baud |
| Power Supply | 3.3V / GND | Power | Stable 3.3V |

Visual Diagram Description

ESP32 #1 (Receiver):

- Connect UART1 pins (GPIO 16 as RX, GPIO 17 as TX) to the SDS011 air quality sensor.

- Connect UART2 pins (GPIO 18 as RX, GPIO 19 as TX) to the SHT20 temperature and humidity sensor (Modbus RTU).

- Connect I2C pins (GPIO 33 as SDA, GPIO 32 as SCL) to the ADP810 pressure sensor.

- Connect SPI pins (CLK = GPIO 14, MOSI = GPIO 13, CS = GPIO 15, RS = GPIO 25, RST = GPIO 26, LED = GPIO 0) to the TFT display.

- Power the ESP32 and all connected sensors using a regulated 3.3V power supply.

ESP32 #2 (Transmitter):

- Connect I2C pins (GPIO 21 as SDA, GPIO 22 as SCL) to the VL53L0X distance sensor.

- Connect UART2 pins (GPIO 16 as RX, GPIO 17 as TX) to the PZEM004T electrical parameter sensor.

- Ensure all components are powered from a stable 3.3V source.

Visual Schematic Suggestion

To create a clear and organized schematic of the system, you can use tools such as Fritzing, KiCad, or EasyEDA. Follow these steps:

- Place two ESP32 Dev Boards on the canvas—label them as ESP32 #1 (Receiver) and ESP32 #2 (Transmitter).

- Add the SDS011 air quality sensor and connect it to UART1 (GPIO 16 and 17) on ESP32 #1.

- Add the SHT20 sensor (Modbus RTU) and connect it to UART2 (GPIO 18 and 19) on ESP32 #1.

- Add the ADP810 pressure sensor and connect it to I2C1 (GPIO 33 as SDA, GPIO 32 as SCL) on ESP32 #1.

- Connect the TFT display to ESP32 #1 using HSPI pins: CLK (GPIO 14), MOSI (GPIO 13), CS (GPIO 15), RS (GPIO 25), RST (GPIO 26), and LED (GPIO 0).

- Place ESP32 #2 and connect:

  - The VL53L0X distance sensor to I2C (GPIO 21 as SDA, GPIO 22 as SCL).

  - The PZEM004T electrical sensor to UART2 (GPIO 16 as RX, GPIO 17 as TX).

- Connect all components to common power and ground rails, ensuring a stable 3.3V regulated power supply.

- Code Explanation

WiFiManager and OTA

- Both ESP32 modules utilize WiFiManager to simplify Wi-Fi configuration. If no credentials are stored, a captive portal is launched for user-friendly network setup.

- Over-the-Air (OTA) update functionality is implemented using the ArduinoOTA library, enabling remote firmware updates without physical access to the devices.

ESP32 #1 (Receiver)

- Hosts a web server on port 80 to receive sensor data via HTTP POST from ESP32 #2.

- Reads particulate matter (PM2.5 and PM10) from the SDS011 sensor using UART communication.

- Acquires temperature and humidity data from the SHT20 sensor via Modbus RTU over UART.

- Retrieves atmospheric pressure readings from the I2C sensor and performs baseline calibration.

- Displays all sensor readings on a TFT display, refreshing every 2 seconds.

- Transmits the aggregated data to cloud services including ThingsBoard (via HTTPS) and a custom Flask API server.

ESP32 #2 (Transmitter)

- Initializes and reads data from the VL53L0X distance sensor and the PZEM004T electrical sensor.

- Measures distance (in cm) and electrical parameters such as voltage, current, power, and energy.

- Sends the collected data to ESP32 #1 every 5 seconds using HTTP POST requests.

- Setup and Usage Instructions

- Assemble the hardware according to the wiring guidelines provided in the documentation.

- Install all required Arduino libraries using the Arduino Library Manager to ensure compatibility with the sensors and modules used.

- Upload the firmware to both ESP32 modules:

  - Flash the Receiver sketch to ESP32 #1.

  - Flash the Transmitter sketch to ESP32 #2.

- On first boot, connect your device (e.g., smartphone or laptop) to the temporary Wi-Fi access points:

  - Kit3-Setup for ESP32 #1

  - Kit3-Transmitter for ESP32 #2

  - Use the captive portal to configure and save your Wi-Fi credentials.

- Once connected to the network, ESP32 #2 will begin transmitting sensor data to ESP32 #1 via HTTP POST.

- ESP32 #1 will display the received data on the TFT screen and forward it to configured cloud services (e.g., ThingsBoard, Flask API).

- Use OTA (Over-the-Air) updates to remotely update the firmware on either ESP32 module as needed.

- Data Flow and Communication

- ESP32 #2 collects distance measurements and electrical parameters (voltage, current, power, and energy).

- It transmits this data to ESP32 #1 via an HTTP POST request to the /update endpoint.

- ESP32 #1 simultaneously reads data from its own connected sensors and merges it with the incoming data from ESP32 #2.

- The combined dataset is then displayed locally on a TFT screen and uploaded to cloud services for remote monitoring and analysis.

- Trouble shooting and FAQs

| Issue | Cause | Solution |
| --- | --- | --- |
| Unable to connect to WiFi | Wrong credentials, weak signal | Reset WiFiManager, relocate device |
| No OTA updates possible | OTA hostname conflict or firewall | Check OTA hostname uniqueness, network |
| No data from SDS011 or SHT20 | Incorrect wiring or baud rate | Verify UART connections and settings |
| TFT display blank or flickering | Wiring error or library mismatch | Verify SPI pins and update TFT library |
| HTTP POST fails to send data | Network/server unreachable | Confirm server IP and network status |

- Code Examples

## CLASS 1 -  ESP1: SHT20 , ESP2: PZEM004T

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <Wire.h>

class SHT20 {

public:

SHT20(TwoWire &wire = Wire) : _wire(wire), _address(0x40) {}

void begin() { _wire.begin(); }

float readTemperature() {

_wire.beginTransmission(_address); _wire.write(0xF3); _wire.endTransmission();

delay(85);

if (_wire.requestFrom(_address, 2) != 2) return NAN;

uint16_t val = (_wire.read() << 8) | _wire.read();

val &= ~0x0003;

return -46.85 + 175.72 * val / 65536.0;

}

float readHumidity() {

_wire.beginTransmission(_address); _wire.write(0xF5); _wire.endTransmission();

delay(29);

if (_wire.requestFrom(_address, 2) != 2) return NAN;

uint16_t val = (_wire.read() << 8) | _wire.read();

val &= ~0x0003;

return -6.0 + 125.0 * val / 65536.0;

}

private:

TwoWire &_wire;

uint8_t _address;

};

WebServer server(80);

String voltage = "N/A";

String current = "N/A";

String power = "N/A";

String energy = "N/A";

SHT20 sht20;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-SHT20")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

sht20.begin();

server.on("/update", HTTP_POST, []() {

voltage = server.arg("voltage");

current = server.arg("current");

power = server.arg("power");

energy = server.arg("energy");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

float temp = sht20.readTemperature();

float hum = sht20.readHumidity();

Serial.printf("Temperature: %.1f C, Humidity: %.1f %%\n", temp, hum);

Serial.printf("Power meter data -> Voltage: %s V, Current: %s A, Power: %s W, Energy: %s Wh\n",

voltage.c_str(), current.c_str(), power.c_str(), energy.c_str());

}

}

ESP2 Code (Transmitter)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <HardwareSerial.h>

#include <PZEM004Tv30.h>

#define PZEM_RX 16

#define PZEM_TX 17

HardwareSerial PZEMSerial(2);

PZEM004Tv30 pzem(PZEMSerial);

const char* serverUrl = "http://192.168.1.3/update";  // Update with ESP1 IP

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-PZEM")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

PZEMSerial.begin(9600, SERIAL_8N1, PZEM_RX, PZEM_TX);

}

void loop() {

ArduinoOTA.handle();

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float energy = pzem.energy();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "voltage=" + String(voltage) +

"&current=" + String(current) +

"&power=" + String(power) +

"&energy=" + String(energy);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(5000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Temperature: 24.1 C, Humidity: 50.0 %

Power meter data -> Voltage: 230.0 V, Current: 0.6 A, Power: 138.0 W, Energy: 10.5 Wh

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS 2 - ESP1: SHT20, ESP2: VL53L0X

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <Wire.h>

class SHT20 {

public:

SHT20(TwoWire &wire = Wire) : _wire(wire), _address(0x40) {}

void begin() { _wire.begin(); }

float readTemperature() {

_wire.beginTransmission(_address); _wire.write(0xF3); _wire.endTransmission();

delay(85);

if (_wire.requestFrom(_address, 2) != 2) return NAN;

uint16_t val = (_wire.read() << 8) | _wire.read();

val &= ~0x0003;

return -46.85 + 175.72 * val / 65536.0;

}

float readHumidity() {

_wire.beginTransmission(_address); _wire.write(0xF5); _wire.endTransmission();

delay(29);

if (_wire.requestFrom(_address, 2) != 2) return NAN;

uint16_t val = (_wire.read() << 8) | _wire.read();

val &= ~0x0003;

return -6.0 + 125.0 * val / 65536.0;

}

private:

TwoWire &_wire;

uint8_t _address;

};

#include <VL53L0X.h>

WebServer server(80);

String distanceStr = "N/A";

SHT20 sht20;

VL53L0X sensor;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-SHT20")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

sht20.begin();

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

server.on("/update", HTTP_POST, []() {

distanceStr = server.arg("distance");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

float temp = sht20.readTemperature();

float hum = sht20.readHumidity();

Serial.printf("Temp: %.1f C, Hum: %.1f %%\n", temp, hum);

Serial.printf("Distance from ESP2: %s mm\n", distanceStr.c_str());

}

}

ESP2 Code (Transmitter)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <Wire.h>

#include <VL53L0X.h>

const char* serverUrl = "http://192.168.1.3/update"; // Update with ESP1 IP

VL53L0X sensor;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-VL53L0X")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

Wire.begin();

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

}

void loop() {

ArduinoOTA.handle();

uint16_t distance = sensor.readRangeContinuousMillimeters();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "distance=" + String(distance);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(2000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Temp: 23.9 C, Hum: 51.0 %

Distance from ESP2: 450 mm

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS 3 - ESP1: SHT20, ESP2: PZEM004T + VL53L0X

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <Wire.h>

class SHT20 {

public:

SHT20(TwoWire &wire = Wire) : _wire(wire), _address(0x40) {}

void begin() { _wire.begin(); }

float readTemperature() {

_wire.beginTransmission(_address); _wire.write(0xF3); _wire.endTransmission();

delay(85);

if (_wire.requestFrom(_address, 2) != 2) return NAN;

uint16_t val = (_wire.read() << 8) | _wire.read();

val &= ~0x0003;

return -46.85 + 175.72 * val / 65536.0;

}

float readHumidity() {

_wire.beginTransmission(_address); _wire.write(0xF5); _wire.endTransmission();

delay(29);

if (_wire.requestFrom(_address, 2) != 2) return NAN;

uint16_t val = (_wire.read() << 8) | _wire.read();

val &= ~0x0003;

return -6.0 + 125.0 * val / 65536.0;

}

private:

TwoWire &_wire;

uint8_t _address;

};

#include <VL53L0X.h>

WebServer server(80);

String voltage = "N/A";

String current = "N/A";

String power = "N/A";

String energy = "N/A";

String distanceStr = "N/A";

SHT20 sht20;

VL53L0X sensor;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-SHT20")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

sht20.begin();

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

server.on("/update", HTTP_POST, []() {

voltage = server.arg("voltage");

current = server.arg("current");

power = server.arg("power");

energy = server.arg("energy");

distanceStr = server.arg("distance");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

float temp = sht20.readTemperature();

float hum = sht20.readHumidity();

Serial.printf("Temp: %.1f C, Hum: %.1f %%\n", temp, hum);

Serial.printf("Power meter -> Volt: %s V, Curr: %s A, Power: %s W, Energy: %s Wh\n",

voltage.c_str(), current.c_str(), power.c_str(), energy.c_str());

Serial.printf("Distance: %s mm\n", distanceStr.c_str());

}

}

ESP2 Code (Transmitter)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <HardwareSerial.h>

#include <PZEM004Tv30.h>

#include <Wire.h>

#include <VL53L0X.h>

#define PZEM_RX 16

#define PZEM_TX 17

HardwareSerial PZEMSerial(2);

PZEM004Tv30 pzem(PZEMSerial);

const char* serverUrl = "http://192.168.1.3/update"; // Update with ESP1 IP

VL53L0X sensor;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-PZEM-VL53")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

PZEMSerial.begin(9600, SERIAL_8N1, PZEM_RX, PZEM_TX);

Wire.begin();

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

}

void loop() {

ArduinoOTA.handle();

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float energy = pzem.energy();

uint16_t distance = sensor.readRangeContinuousMillimeters();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "voltage=" + String(voltage) +

"&current=" + String(current) +

"&power=" + String(power) +

"&energy=" + String(energy) +

"&distance=" + String(distance);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(5000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Temp: 24.0 C, Hum: 52.0 %

Power meter -> Volt: 229.9 V, Curr: 0.7 A, Power: 161.0 W, Energy: 11.0 Wh

Distance: 448 mm

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS 4 - ESP1: ADP810, ESP2: PZEM004T

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <Wire.h>

class ADP810 {

public:

ADP810(TwoWire &wire = Wire, uint8_t addr = 0x25) : _wire(wire), _address(addr) {}

void begin() { _wire.begin(); }

int16_t readPressure() {

_wire.beginTransmission(_address);

_wire.write(0x00);

if (_wire.endTransmission(false) != 0) return -1;

if (_wire.requestFrom(_address, (uint8_t)2) != 2) return -1;

uint16_t data = (_wire.read() << 8) | _wire.read();

return (int16_t)data;

}

private:

TwoWire &_wire;

uint8_t _address;

};

WebServer server(80);

String voltage = "N/A";

String current = "N/A";

String power = "N/A";

String energy = "N/A";

ADP810 adp810;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-ADP810")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

adp810.begin();

server.on("/update", HTTP_POST, []() {

voltage = server.arg("voltage");

current = server.arg("current");

power = server.arg("power");

energy = server.arg("energy");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

int16_t pressure = adp810.readPressure();

Serial.printf("Pressure: %d\n", pressure);

Serial.printf("Power meter -> Volt: %s V, Curr: %s A, Power: %s W, Energy: %s Wh\n",

voltage.c_str(), current.c_str(), power.c_str(), energy.c_str());

}

}

ESP2 Code (Transmitter)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <HardwareSerial.h>

#include <PZEM004Tv30.h>

#define PZEM_RX 16

#define PZEM_TX 17

HardwareSerial PZEMSerial(2);

PZEM004Tv30 pzem(PZEMSerial);

const char* serverUrl = "http://192.168.1.3/update";  // Update with ESP1 IP

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-PZEM")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

PZEMSerial.begin(9600, SERIAL_8N1, PZEM_RX, PZEM_TX);

}

void loop() {

ArduinoOTA.handle();

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float energy = pzem.energy();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "voltage=" + String(voltage) +

"&current=" + String(current) +

"&power=" + String(power) +

"&energy=" + String(energy);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(5000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Pressure: 1012

Power meter -> Volt: 229.7 V, Curr: 0.8 A, Power: 183.0 W, Energy: 12.3 Wh

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS #5 - ESP1: ADP810, ESP2: VL53L0X

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <Wire.h>

class ADP810 {

public:

ADP810(TwoWire &wire = Wire, uint8_t addr = 0x25) : _wire(wire), _address(addr) {}

void begin() { _wire.begin(); }

int16_t readPressure() {

_wire.beginTransmission(_address);

_wire.write(0x00);

if (_wire.endTransmission(false) != 0) return -1;

if (_wire.requestFrom(_address, (uint8_t)2) != 2) return -1;

uint16_t data = (_wire.read() << 8) | _wire.read();

return (int16_t)data;

}

private:

TwoWire &_wire;

uint8_t _address;

};

#include <VL53L0X.h>

WebServer server(80);

String distanceStr = "N/A";

ADP810 adp810;

VL53L0X sensor;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-ADP810")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

adp810.begin();

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

server.on("/update", HTTP_POST, []() {

distanceStr = server.arg("distance");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

int16_t pressure = adp810.readPressure();

Serial.printf("Pressure: %d\n", pressure);

Serial.printf("Distance from ESP2: %s mm\n", distanceStr.c_str());

}

}

ESP2 Code (Transmitter)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <Wire.h>

#include <VL53L0X.h>

const char* serverUrl = "http://192.168.1.3/update"; // Update with ESP1 IP

VL53L0X sensor;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-VL53L0X")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

Wire.begin();

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

}

void loop() {

ArduinoOTA.handle();

uint16_t distance = sensor.readRangeContinuousMillimeters();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "distance=" + String(distance);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(2000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Pressure: 1013

Distance from ESP2: 452 mm

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS 6 - ESP1: TFT, ESP2: PZEM004T

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <TFT_eSPI.h>

TFT_eSPI tft = TFT_eSPI();

WebServer server(80);

String voltage = "N/A";

String current = "N/A";

String power = "N/A";

String energy = "N/A";

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-TFT")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE, TFT_BLACK);

tft.setTextSize(2);

server.on("/update", HTTP_POST, []() {

voltage = server.arg("voltage");

current = server.arg("current");

power = server.arg("power");

energy = server.arg("energy");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

tft.fillScreen(TFT_BLACK);

tft.setCursor(0, 0);

tft.printf("Voltage: %s V\n", voltage.c_str());

tft.printf("Current: %s A\n", current.c_str());

tft.printf("Power: %s W\n", power.c_str());

tft.printf("Energy: %s Wh\n", energy.c_str());

Serial.printf("Power meter -> Volt: %s V, Curr: %s A, Power: %s W, Energy: %s Wh\n",

voltage.c_str(), current.c_str(), power.c_str(), energy.c_str());

}

}

ESP2 Code (Transmitter)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <HardwareSerial.h>

#include <PZEM004Tv30.h>

#define PZEM_RX 16

#define PZEM_TX 17

HardwareSerial PZEMSerial(2);

PZEM004Tv30 pzem(PZEMSerial);

const char* serverUrl = "http://192.168.1.3/update";  // Update with ESP1 IP

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-PZEM")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

PZEMSerial.begin(9600, SERIAL_8N1, PZEM_RX, PZEM_TX);

}

void loop() {

ArduinoOTA.handle();

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float energy = pzem.energy();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "voltage=" + String(voltage) +

"&current=" + String(current) +

"&power=" + String(power) +

"&energy=" + String(energy);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(5000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Power meter -> Volt: 230.0 V, Curr: 0.9 A, Power: 207.0 W, Energy: 13.5 Wh

On TFT display:

Voltage: 230.0 V

Current: 0.9 A

Power: 207.0 W

Energy: 13.5 Wh

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS 7 - ESP1: TFT, ESP2: VL53L0X

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <TFT_eSPI.h>

#include <VL53L0X.h>

TFT_eSPI tft = TFT_eSPI();

VL53L0X sensor;

WebServer server(80);

String distanceStr = "N/A";

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-TFT")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE, TFT_BLACK);

tft.setTextSize(2);

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

server.on("/update", HTTP_POST, []() {

distanceStr = server.arg("distance");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

tft.fillScreen(TFT_BLACK);

tft.setCursor(0, 0);

tft.printf("Distance: %s mm\n", distanceStr.c_str());

Serial.printf("Distance from ESP2: %s mm\n", distanceStr.c_str());

}

}

ESP2 Code (Transmitter)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <Wire.h>

#include <VL53L0X.h>

const char* serverUrl = "http://192.168.1.3/update"; // Update with ESP1 IP

VL53L0X sensor;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-VL53L0X")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

Wire.begin();

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

}

void loop() {

ArduinoOTA.handle();

uint16_t distance = sensor.readRangeContinuousMillimeters();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "distance=" + String(distance);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(2000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Distance from ESP2: 447 mm

On TFT display:

Distance: 447 mm

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS 8 - ESP1: ADP810 + TFT,  ESP2: PZEM004T

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <Wire.h>

#include <TFT_eSPI.h>

class ADP810 {

public:

ADP810(TwoWire &wire = Wire, uint8_t addr = 0x25) : _wire(wire), _address(addr) {}

void begin() { _wire.begin(); }

int16_t readPressure() {

_wire.beginTransmission(_address);

_wire.write(0x00);

if (_wire.endTransmission(false) != 0) return -1;

if (_wire.requestFrom(_address, (uint8_t)2) != 2) return -1;

uint16_t data = (_wire.read() << 8) | _wire.read();

return (int16_t)data;

}

private:

TwoWire &_wire;

uint8_t _address;

};

TFT_eSPI tft = TFT_eSPI();

WebServer server(80);

String voltage = "N/A";

String current = "N/A";

String power = "N/A";

String energy = "N/A";

ADP810 adp810;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-ADP810-TFT")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

adp810.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE, TFT_BLACK);

tft.setTextSize(2);

server.on("/update", HTTP_POST, []() {

voltage = server.arg("voltage");

current = server.arg("current");

power = server.arg("power");

energy = server.arg("energy");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

int16_t pressure = adp810.readPressure();

tft.fillScreen(TFT_BLACK);

tft.setCursor(0, 0);

tft.printf("Pressure: %d\n", pressure);

tft.printf("Voltage: %s V\n", voltage.c_str());

tft.printf("Current: %s A\n", current.c_str());

tft.printf("Power: %s W\n", power.c_str());

tft.printf("Energy: %s Wh\n", energy.c_str());

Serial.printf("Pressure: %d\n", pressure);

Serial.printf("Power meter -> Volt: %s V, Curr: %s A, Power: %s W, Energy: %s Wh\n",

voltage.c_str(), current.c_str(), power.c_str(), energy.c_str());

}

}

ESP2 Code (Transmitter)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <HardwareSerial.h>

#include <PZEM004Tv30.h>

#define PZEM_RX 16

#define PZEM_TX 17

HardwareSerial PZEMSerial(2);

PZEM004Tv30 pzem(PZEMSerial);

const char* serverUrl = "http://192.168.1.3/update";  // Update with ESP1 IP

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-PZEM")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

PZEMSerial.begin(9600, SERIAL_8N1, PZEM_RX, PZEM_TX);

}

void loop() {

ArduinoOTA.handle();

float voltage = pzem.voltage();

float current = pzem.current();

float power = pzem.power();

float energy = pzem.energy();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "voltage=" + String(voltage) +

"&current=" + String(current) +

"&power=" + String(power) +

"&energy=" + String(energy);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(5000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Pressure: 1015

Power meter -> Volt: 230.1 V, Curr: 0.9 A, Power: 207.1 W, Energy: 13.8 Wh

On TFT display:

Pressure: 1015

Voltage: 230.1 V

Current: 0.9 A

Power: 207.1 W

Energy: 13.8 Wh

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS 9 - ESP1: ADP810 + TFT, ESP2: VL53L0X

ESP1 Code (Receiver)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <Wire.h>

#include <TFT_eSPI.h>

#include <VL53L0X.h>

class ADP810 {

public:

ADP810(TwoWire &wire = Wire, uint8_t addr = 0x25) : _wire(wire), _address(addr) {}

void begin() { _wire.begin(); }

int16_t readPressure() {

_wire.beginTransmission(_address);

_wire.write(0x00);

if (_wire.endTransmission(false) != 0) return -1;

if (_wire.requestFrom(_address, (uint8_t)2) != 2) return -1;

uint16_t data = (_wire.read() << 8) | _wire.read();

return (int16_t)data;

}

private:

TwoWire &_wire;

uint8_t _address;

};

TFT_eSPI tft = TFT_eSPI();

VL53L0X sensor;

WebServer server(80);

String distanceStr = "N/A";

ADP810 adp810;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP1-ADP810-TFT")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

adp810.begin();

tft.init();

tft.setRotation(1);

tft.fillScreen(TFT_BLACK);

tft.setTextColor(TFT_WHITE, TFT_BLACK);

tft.setTextSize(2);

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

server.on("/update", HTTP_POST, []() {

distanceStr = server.arg("distance");

server.send(200, "text/plain", "OK");

});

server.begin();

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis > 2000) {

lastMillis = millis();

int16_t pressure = adp810.readPressure();

tft.fillScreen(TFT_BLACK);

tft.setCursor(0, 0);

tft.printf("Pressure: %d\n", pressure);

tft.printf("Distance: %s mm\n", distanceStr.c_str());

Serial.printf("Pressure: %d\n", pressure);

Serial.printf("Distance from ESP2: %s mm\n", distanceStr.c_str());

}

}

ESP2 TRANSMITTER:

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <Wire.h>

#include <VL53L0X.h>

const char* serverUrl = "http://192.168.1.3/update"; // Update with ESP1 IP

VL53L0X sensor;

void setup() {

Serial.begin(115200);

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("ESP2-VL53L0X")) ESP.restart();

Serial.println("WiFi connected: " + WiFi.localIP().toString());

ArduinoOTA.begin();

Wire.begin();

sensor.init();

sensor.setTimeout(500);

sensor.startContinuous();

}

void loop() {

ArduinoOTA.handle();

uint16_t distance = sensor.readRangeContinuousMillimeters();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "distance=" + String(distance);

int code = http.POST(postData);

Serial.printf("POST response: %d\n", code);

http.end();

}

delay(2000);

}

ESP1 Serial Output

WiFi connected: 192.168.X.X

Pressure: 1016

Distance from ESP2: 450 mm

On TFT display:

Pressure: 1016

Distance: 450 mm

ESP2 Serial Output

WiFi connected: 192.168.X.X

POST response: 200

## CLASS 10 – Main Code

ESP1(RECEIVER CODE)

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <WebServer.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <ArduinoJson.h>

#include <Wire.h>

#include <ModbusMaster.h>

#include <SPI.h>

#include "TFT_22_ILI9225.h"

#include <../fonts/FreeSans9pt7b.h>

// === TFT Display Pins ===

#define TFT_RST 26

#define TFT_RS  25

#define TFT_CS  15

#define TFT_SDI 13

#define TFT_CLK 14

#define TFT_LED 0

#define TFT_BRIGHTNESS 200

SPIClass hspi(HSPI);

TFT_22_ILI9225 tft = TFT_22_ILI9225(TFT_RST, TFT_RS, TFT_CS, TFT_LED, TFT_BRIGHTNESS);

// === SDS011 (UART1) PM sensor ===

#define SDS_RX 16

#define SDS_TX 17

HardwareSerial SerialSDS(1);

float pm25 = 0, pm10 = 0;

// === SHT20 Modbus (UART2) ===

#define RXD2 18

#define TXD2 19

HardwareSerial SerialSHT(2);

ModbusMaster sht20;

// === ADP810 Pressure Sensor (I2C2) ===

#define SDA_ADP 33

#define SCL_ADP 32

#define ADP810_ADDR 0x25

TwoWire WireADP = TwoWire(1);

long baselinePressure = 0;

// === ESP32 #2 Data from WebServer ===

WebServer server(80);

String distStr = "N/A", voltStr = "N/A", currStr = "N/A", powerStr = "N/A", energyStr = "N/A";

// === Cloud APIs ===

const char* kitID = "kit3";

const char* thingsboardURL = "https://192.168.1.238:8080/api/v1/esp3/telemetry";

const char* flaskAPI = "http://192.168.1.130:5010/api/v1/data";

void setup() {

Serial.begin(115200);

// WiFiManager

WiFiManager wm;

wm.setTimeout(180);

if (!wm.autoConnect("Kit3-Setup")) {

Serial.println("Failed to connect WiFi. Restarting...");

ESP.restart();

}

Serial.println("WiFi connected: " + WiFi.localIP().toString());

// OTA

ArduinoOTA.setHostname("kit3");

ArduinoOTA.begin();

// Web Server for ESP32 #2 data

server.on("/update", HTTP_POST, []() {

distStr = server.arg("distance");

voltStr = server.arg("voltage");

currStr = server.arg("current");

powerStr = server.arg("power");

energyStr = server.arg("energy");

Serial.println("Received data from ESP32 #2");

server.send(200, "text/plain", "OK");

});

server.begin();

// SDS011 setup

SerialSDS.begin(9600, SERIAL_8N1, SDS_RX, SDS_TX);

// SHT20 Modbus setup

SerialSHT.begin(9600, SERIAL_8N1, RXD2, TXD2);

sht20.begin(1, SerialSHT);

// ADP810 I2C2 setup

WireADP.begin(SDA_ADP, SCL_ADP);

calibrateADP810();

// TFT setup

hspi.begin(TFT_CLK, -1, TFT_SDI, TFT_CS);

tft.begin(hspi);

tft.setOrientation(0);

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 20, "Sensor Monitor", COLOR_YELLOW);

}

void loop() {

ArduinoOTA.handle();

server.handleClient();

static unsigned long lastMillis = 0;

if (millis() - lastMillis >= 2000) {

lastMillis = millis();

// Read SDS011 PM sensor

if (readSDS011()) {

// Apply correction factors

pm25 *= 0.79;

pm10 *= 1.61;

}

// Read SHT20 temperature and humidity via Modbus

if (sht20.readInputRegisters(0x0001, 2) == sht20.ku8MBSuccess) {

float temp = sht20.getResponseBuffer(0) / 100.0;

float hum = sht20.getResponseBuffer(1) / 100.0;

float pressure = readADP810Pressure();

updateDisplay(temp, hum, pressure);

sendToCloud(temp, hum, pressure, pm25, pm10);

}

}

}

bool readSDS011() {

static uint8_t buffer[10];

static uint8_t idx = 0;

while (SerialSDS.available()) {

uint8_t c = SerialSDS.read();

if (idx == 0 && c != 0xAA) continue;

buffer[idx++] = c;

if (idx == 10) {

idx = 0;

if (buffer[0] == 0xAA && buffer[9] == 0xAB) {

pm25 = ((buffer[3] << 8) + buffer[2]) / 10.0;

pm10 = ((buffer[5] << 8) + buffer[4]) / 10.0;

return true;

}

}

}

return false;

}

void updateDisplay(float temp, float hum, float pressure) {

tft.clear();

tft.setGFXFont(&FreeSans9pt7b);

tft.drawGFXText(10, 20, "Sensor Monitor", COLOR_YELLOW);

int16_t labelX = 5, valueX = 85, y = 45, lineSpacing = 22;

tft.drawGFXText(labelX, y, "PM2.5:", COLOR_GREEN);

tft.drawGFXText(valueX, y, String(pm25, 1) + " ug", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "PM10:", COLOR_GREEN);

tft.drawGFXText(valueX, y, String(pm10, 1) + " ug", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Temp:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(temp, 1) + " C", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Humid:", COLOR_CYAN);

tft.drawGFXText(valueX, y, String(hum, 1) + " %", COLOR_WHITE); y += lineSpacing;

float pres = readADP810Pressure();

tft.drawGFXText(labelX, y, "Press:", COLOR_ORANGE);

tft.drawGFXText(valueX, y, String(pres, 1) + " Pa", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Dist:", COLOR_MAGENTA);

tft.drawGFXText(valueX, y, distStr + " cm", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Volt:", COLOR_MAGENTA);

tft.drawGFXText(valueX, y, voltStr + " V", COLOR_WHITE); y += lineSpacing;

tft.drawGFXText(labelX, y, "Curr:", COLOR_MAGENTA);

tft.drawGFXText(valueX, y, currStr + " A", COLOR_WHITE); y += lineSpacing;

// tft.drawGFXText(labelX, y, "Power:", COLOR_MAGENTA);

//tft.drawGFXText(valueX, y, powerStr + " W", COLOR_WHITE); y += lineSpacing;

}

void sendToCloud(float temp, float hum, float pressure, float pm25, float pm10) {

// ThingsBoard POST

WiFiClientSecure client;

client.setInsecure();  // Warning: insecure connection, replace with proper cert in prod

HTTPClient https;

if (https.begin(client, thingsboardURL)) {

https.addHeader("Content-Type", "application/json");

StaticJsonDocument<256> doc;

doc["temperature"] = temp;

doc["humidity"] = hum;

doc["pressure"] = pressure;

doc["pm2_5"] = pm25;

doc["pm10"] = pm10;

doc["distance"] = distStr.toFloat();

doc["voltage"] = voltStr.toFloat();

doc["current"] = currStr.toFloat();

doc["power"] = powerStr.toFloat();

doc["energy"] = energyStr.toFloat();

String json;

serializeJson(doc, json);

int code = https.POST(json);

Serial.printf("ThingsBoard POST code: %d\n", code);

https.end();

} else {

Serial.println("Failed to connect to ThingsBoard");

}

// Flask API POST

WiFiClient httpClient;

HTTPClient http;

if (http.begin(httpClient, flaskAPI)) {

http.addHeader("Content-Type", "application/json");

StaticJsonDocument<300> doc;

doc["kit_id"] = kitID;

doc["temperature"] = temp;

doc["humidity"] = hum;

doc["pressure"] = pressure;

doc["pm2_5"] = pm25;

doc["pm10"] = pm10;

doc["distance"] = distStr.toFloat();

doc["voltage"] = voltStr.toFloat();

doc["current"] = currStr.toFloat();

doc["power"] = powerStr.toFloat();

doc["energy"] = energyStr.toFloat();

String json;

serializeJson(doc, json);

int code = http.POST(json);

Serial.printf("Flask POST code: %d\n", code);

http.end();

} else {

Serial.println("Failed to connect to Flask API");

}

}

// ADP810 sensor calibration

void calibrateADP810() {

long sum = 0;

for (int i = 0; i < 20; i++) {

sum += readADP810PressureRaw();

delay(10);

}

baselinePressure = sum / 20;

}

// Read raw ADP810 pressure

long readADP810PressureRaw() {

WireADP.beginTransmission(ADP810_ADDR);

WireADP.write(0xF8);

if (WireADP.endTransmission(false) != 0) return -1;

WireADP.requestFrom(ADP810_ADDR, (uint8_t)2);

if (WireADP.available() < 2) return -1;

uint8_t msb = WireADP.read();

uint8_t lsb = WireADP.read();

return ((long)msb << 8) | lsb;

}

float readADP810Pressure() {

long raw = readADP810PressureRaw();

if (raw < 0) return -1;

long corrected = raw - baselinePressure;

// Convert to Pascals (example factor, adjust to your sensor)

return corrected * 0.0345;

}

ESP2 ( TRANSMITTER CODE )

#include <WiFi.h>

#include <WiFiManager.h>

#include <ArduinoOTA.h>

#include <HTTPClient.h>

#include <Wire.h>

#include <Adafruit_VL53L0X.h>

#include <PZEM004Tv30.h>

// VL53L0X

Adafruit_VL53L0X lox = Adafruit_VL53L0X();

// PZEM

#define PZEM_RX_PIN 16

#define PZEM_TX_PIN 17

HardwareSerial PZEMSerial(2);

PZEM004Tv30 pzem(PZEMSerial, PZEM_RX_PIN, PZEM_TX_PIN);

// Replace with your server URL (ESP32 #1 IP)

const char* serverUrl = "http://192.168.1.3/update";

void setup() {

Serial.begin(115200);

// WiFiManager to handle WiFi connection + captive portal

WiFiManager wm;

wm.setTimeout(180); // 3 minutes timeout for config portal

if (!wm.autoConnect("Kit3-Transmitter")) {

Serial.println("Failed to connect or configure WiFi. Restarting...");

ESP.restart();

}

Serial.println("WiFi connected: " + WiFi.localIP().toString());

// Setup ArduinoOTA

ArduinoOTA.setHostname("Kit3-Transmitter");

ArduinoOTA.begin();

// VL53L0X sensor init

Wire.begin(21, 22);

if (!lox.begin()) {

Serial.println("VL53L0X init failed");

while (1) delay(1000);

}

// PZEM sensor init

PZEMSerial.begin(9600, SERIAL_8N1, PZEM_RX_PIN, PZEM_TX_PIN);

}

void loop() {

ArduinoOTA.handle();

VL53L0X_RangingMeasurementData_t measure;

lox.rangingTest(&measure, false);

float distance_cm = measure.RangeStatus != 4 ? measure.RangeMilliMeter / 10.0 : -1;

float voltage = pzem.voltage();

float current = pzem.current();

float power   = pzem.power();

float energy  = pzem.energy();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverUrl);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String postData = "distance=" + String(distance_cm) +

"&voltage=" + String(voltage) +

"&current=" + String(current) +

"&power=" + String(power) +

"&energy=" + String(energy);

int responseCode = http.POST(postData);

if (responseCode > 0)

Serial.println("Data sent: " + postData);

else

Serial.println("HTTP error: " + http.errorToString(responseCode));

http.end();

} else {

Serial.println("WiFi disconnected");

}

delay(5000);  // Send data every 5 seconds

}

ESP32 #1 (Receiver + Display + Sensors) Sample Serial Output

WiFi connected: 192.168.1.100

Received data from ESP32 #2

ThingsBoard POST code: 200

Flask POST code: 200

Explanation:

WiFi connected with IP shown.

When ESP32 #2 sends data (distance, voltage, current, power, energy), it prints "Received data from ESP32 #2".

After sending JSON to ThingsBoard and Flask APIs, prints HTTP response codes (usually 200 if success).

ESP32 #2 (Transmitter) Sample Serial Output

WiFi connected: 192.168.1.101

Data sent: distance=45.3&voltage=230.1&current=0.58&power=134.3&energy=10.6

Data sent: distance=44.9&voltage=230.2&current=0.59&power=135.5&energy=10.7

Data sent: distance=45.1&voltage=230.0&current=0.57&power=132.9&energy=10.5

Explanation:

Shows WiFi IP on startup.

Every 5 seconds sends data to ESP32 #1 webserver.

Prints the POST data it sends.

If WiFi disconnected, it would print "WiFi disconnected".

Sample TFT Display Output :

Sensor Monitor

PM2.5:    12.3 ug

PM10:     25.7 ug

Temp:     23.4 C

Humid:    45.6 %

Press:    1013.2 Pa

Dist:     45.3 cm

Volt:     230.1

Curr:     0.58 A